ICEngineSoft Test Report

Report Date : Wed, Feb 01, 2017 , Time :  2:02:38 PM

Organization : Organisation, Operator : Operator

Data File : T10.CR 17.5.xlsx, Last Modified Date: Wed, Feb 01, 2017,

Config. File : VCR Diesel Engine.xls

Engine Details :

ICEngine set up under test is Research Diesel having power 3.50 kW @ 1500 rpm which is 1 Cylinder, Four stroke , Constant Speed, Water Cooled, Diesel Engine, with Cylinder Bore 87.50(mm), Stroke Length 110.00(mm), Connecting Rod length 234.00(mm), Compression Ratio 17.50, Swept volume 661.45 (cc)

Combustion Parameters :

Specific Gas Const (kJ/kgK) : 1.00, Air Density (kg/m^3) : 1.17, Adiabatic Index : 1.41, Polytrophic Index : -0.00, Number Of Cycles : 10, Cylinder Pressure Referance :  1, Smoothing 2, TDC Reference : 0

Performance Parameters :

Orifice Diameter (mm) : 20.00, Orifice Coeff. Of Discharge : 0.60, Dynamometer Arm Legnth (mm) : 185, Fuel Pipe dia (mm) : 12.40, Ambient Temp. (Deg C) : 27, Pulses Per revolution : 360, Fuel Type : Diesel, Fuel Density (Kg/m^3) : 830, Calorific Value Of Fuel (kj/kg) : 42000

IP, BP & FP

| Speed (rpm) | Load (kg) | IP (kW) | BP (kW) | FP (kW) |
| --- | --- | --- | --- | --- |
| 1563.00 | 0.18 | 2.67 | 0.05 | 2.62 |
| 1547.00 | 2.09 | 1.76 | 0.61 | 1.15 |
| 1534.00 | 4.10 | 2.32 | 1.19 | 1.13 |
| 1523.00 | 6.06 | 3.14 | 1.75 | 1.39 |
| 1516.00 | 8.02 | 4.15 | 2.31 | 1.84 |

IMEP,BMEP & FMEP

| Speed (rpm) | Load (kg) | IMEP (bar) | BMEP (bar) | FMEP (bar) |
| --- | --- | --- | --- | --- |
| 1563.00 | 0.18 | 3.10 | 0.06 | 3.04 |
| 1547.00 | 2.09 | 2.07 | 0.72 | 1.35 |
| 1534.00 | 4.10 | 2.75 | 1.41 | 1.34 |
| 1523.00 | 6.06 | 3.74 | 2.09 | 1.65 |
| 1516.00 | 8.02 | 4.96 | 2.77 | 2.20 |

Air &  Fuel Flow

| Speed (rpm) | Load (kg) | Air (mmWC) | Fuel (cc/min) |
| --- | --- | --- | --- |
| 1563.00 | 0.18 | 85.87 | 9.00 |
| 1547.00 | 2.09 | 80.86 | 12.00 |
| 1534.00 | 4.10 | 78.56 | 14.00 |
| 1523.00 | 6.06 | 76.28 | 15.00 |
| 1516.00 | 8.02 | 77.06 | 18.00 |

Indicated & Brake Thermal Efficiency

| Speed (rpm) | Load (kg) | IThEff (%) | BThEff (%) |
| --- | --- | --- | --- |
| 1563.00 | 0.18 | 51.10 | 1.04 |
| 1547.00 | 2.09 | 25.31 | 8.81 |
| 1534.00 | 4.10 | 28.58 | 14.69 |
| 1523.00 | 6.06 | 36.03 | 20.13 |
| 1516.00 | 8.02 | 39.64 | 22.10 |

SFC & Fuel Consumption

| Speed (rpm) | Load (kg) | SFC (kg/kWh) | Fuel (kg/h) |
| --- | --- | --- | --- |
| 1563.00 | 0.18 | 8.26 | 0.45 |
| 1547.00 | 2.09 | 0.97 | 0.60 |
| 1534.00 | 4.10 | 0.58 | 0.70 |
| 1523.00 | 6.06 | 0.43 | 0.75 |
| 1516.00 | 8.02 | 0.39 | 0.90 |

TORQUE, Mechanical & Volmertic Efficiency

| Speed (rpm) | Load (Kg) | Torque (Nm) | Mech Eff. (%) | Vol Eff. (%) |
| --- | --- | --- | --- | --- |
| 1563.00 | 0.18 | 0.33 | 2.03 | 82.89 |
| 1547.00 | 2.09 | 3.79 | 34.81 | 81.26 |
| 1534.00 | 4.10 | 7.44 | 51.41 | 80.78 |
| 1523.00 | 6.06 | 11.00 | 55.87 | 80.17 |
| 1516.00 | 8.02 | 14.56 | 55.74 | 80.95 |

HBP,HJW & HGas

| Speed (rpm) | Load (kg) | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- |
| 1563.00 | 0.18 | 1.04 | 31.76 | 27.10 | 40.10 |
| 1547.00 | 2.09 | 8.81 | 54.47 | 28.14 | 8.59 |
| 1534.00 | 4.10 | 14.69 | 63.15 | 27.51 | 0.00 |
| 1523.00 | 6.06 | 20.13 | 73.48 | 29.61 | 0.00 |
| 1516.00 | 8.02 | 22.10 | 46.40 | 28.78 | 2.72 |

Observation Data

| Speed (rpm) | Load (kg) | Comp Ratio | T1 (deg C) | T2 (deg C) | T3 (deg C) | T4 (deg C) | T5 (deg C) | T6 (deg C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1563 | 0.18 | 17.50 | 24.72 | 29.48 | 24.73 | 27.62 | 165.82 | 138.97 |
| 1547 | 2.09 | 17.50 | 25.23 | 36.12 | 25.24 | 33.90 | 223.93 | 186.70 |
| 1534 | 4.10 | 17.50 | 25.56 | 40.29 | 25.57 | 38.06 | 254.07 | 212.09 |
| 1523 | 6.06 | 17.50 | 25.88 | 44.24 | 25.88 | 42.55 | 292.19 | 241.84 |
| 1516 | 8.02 | 17.50 | 26.41 | 40.32 | 26.41 | 40.27 | 333.23 | 277.79 |

Observation Data

| Air (mmWC) | Fuel (cc/min) | WaterFlow Engine (lph) | WaterFlow Cal (lph) |
| --- | --- | --- | --- |
| 85.87 | 9.00 | 300 | 100 |
| 80.86 | 12.00 | 300 | 100 |
| 78.56 | 14.00 | 300 | 100 |
| 76.28 | 15.00 | 300 | 100 |
| 77.06 | 18.00 | 300 | 100 |

Result Data

| Torque (Nm) | BP (kW) | FP (kW) | IP (kW) | BMEP (bar) | IMEP (bar) | BTHE (%) | ITHE (%) | Mech Eff. (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.33 | 0.05 | 2.62 | 2.67 | 0.06 | 3.10 | 1.04 | 51.10 | 2.03 |
| 3.79 | 0.61 | 1.15 | 1.76 | 0.72 | 2.07 | 8.81 | 25.31 | 34.81 |
| 7.44 | 1.19 | 1.13 | 2.32 | 1.41 | 2.75 | 14.69 | 28.58 | 51.41 |
| 11.00 | 1.75 | 1.39 | 3.14 | 2.09 | 3.74 | 20.13 | 36.03 | 55.87 |
| 14.56 | 2.31 | 1.84 | 4.15 | 2.77 | 4.96 | 22.10 | 39.64 | 55.74 |

Result Data

| Air Flow (kg/h) | Fuel Flow (kg/h) | SFC (kg/kWh) | Vol Eff. (%) | A/F Ratio | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30.18 | 0.45 | 8.26 | 82.89 | 67.33 | 1.04 | 31.76 | 27.10 | 40.10 |
| 29.28 | 0.60 | 0.97 | 81.26 | 49.00 | 8.81 | 54.47 | 28.14 | 8.59 |
| 28.87 | 0.70 | 0.58 | 80.78 | 41.40 | 14.69 | 63.15 | 27.51 | 0.00 |
| 28.44 | 0.75 | 0.43 | 80.17 | 38.08 | 20.13 | 73.48 | 29.61 | 0.00 |
| 28.59 | 0.90 | 0.39 | 80.95 | 31.89 | 22.10 | 46.40 | 28.78 | 2.72 |