Experiment No 1

Constructional details of I.C. engines by dismantling and assembly.

AIM- Constructional detail of I.C. engines by dismantling and assembly.

DESCRPITION :

The internal combustion engine is an engine in which the combustion of a fuel occurs with an oxidizer (usually air) in a combustion chamber . In an internal combustion engine the expansion of the high temperature and pressure gases, which are produced by the combustion, directly applies force to a movable component of the engine( pistons) by moving it over a distance, generate useful mechanical energy.

All internal combustion engines depend on the exothermic chemical process of combustion . The reaction of a fuel , typically with oxygen from the air. The combustion process typically results in the production of a great quantity of heat, as well as the production of steam and carbon dioxide and other chemicals at very high temperature, the temperature reached is determined by the chemical make up of the fuel and oxidizers.

The principle behind any reciprocating internal combustion engine: If you put a tiny amount of high-energy fuel (like gasoline) in a small, enclosed space and ignite it, an incredible amount of energy is released in the form of expanding gas.

Classification of I.C. Engines:

Internal combustion engines may be classified as given below:

1. According to cycle of operation:

• Two-stroke cycle engines

• Four-stroke cycle engines

2. According to cycle of combustion:

• Otto cycle engine

• Diesel cycle engine

• Dual-combustion

3. According to the fuel employed and the method of fuel supply to the engine

cylinder:

• Petrol engine

• Diesel engine

• Oil, Gas engine

4. According to method of ignition:

• · Spark ignition (S.L) engine

• · Compression ignition (C.I.) engine

5. According to method of cooling the cylinder:

• Air-cooled engine

• Water-cooled engine

6. According to number of cylinders:

• Single cylinder engine

• Multi-cylinder engine

Different Parts of I.C. Engines :

1. Cylinder Block

2. Cylinder

3. Piston

4. Combustion Chamber

5. Inlet manifold

6. Exhaust Manifold

7. Inlet and Exhaust Valves

8. Spark Plug

9. Connecting rod

10.Crankshaft

11. Piston Rings

12. Gudgeon Pin

13. CamShaft

14. Cams

15. Flywheel

CYLINDER BLOCK

PISTON & RINGS

CONNECTING ROD

INLET MANIFOLD

FLYWHEEL

SPARK PLUG