ICEngineSoft Test Report

Report Date : Tue, Jan 23, 2018 , Time :  11:43:23 AM

Organization : Organisation, Operator : Operator

Data File : T1.CR17.xlsx, Last Modified Date: Tue, Jan 23, 2018,

Config. File : VCR Diesel Engine.xls

Engine Details :

ICEngine set up under test is Research Diesel having power 3.50 kW @ 1500 rpm which is 1 Cylinder, Four stroke , Constant Speed, Water Cooled, Diesel Engine, with Cylinder Bore 87.50(mm), Stroke Length 110.00(mm), Connecting Rod length 234.00(mm), Compression Ratio 17.00, Swept volume 661.45 (cc)

Combustion Parameters :

Specific Gas Const (kJ/kgK) : 1.00, Air Density (kg/m^3) : 1.17, Adiabatic Index : 1.41, Polytrophic Index : -0.00, Number Of Cycles : 10, Cylinder Pressure Referance :  0, Smoothing 2, TDC Reference : 0

Performance Parameters :

Orifice Diameter (mm) : 20.00, Orifice Coeff. Of Discharge : 0.60, Dynamometer Arm Legnth (mm) : 185, Fuel Pipe dia (mm) : 12.40, Ambient Temp. (Deg C) : 27, Pulses Per revolution : 360, Fuel Type : Diesel, Fuel Density (Kg/m^3) : 830, Calorific Value Of Fuel (kj/kg) : 42000

IP, BP & FP

| Speed (rpm) | Load (kg) | IP (kW) | BP (kW) | FP (kW) |
| --- | --- | --- | --- | --- |
| 1567.00 | 0.32 | 3.27 | 0.09 | 3.17 |
| 1538.00 | 2.63 | 3.15 | 0.77 | 2.38 |
| 1527.00 | 4.54 | 3.16 | 1.32 | 1.84 |
| 1520.00 | 6.24 | 3.13 | 1.80 | 1.32 |
| 1518.00 | 7.05 | 2.69 | 2.03 | 0.65 |

IMEP,BMEP & FMEP

| Speed (rpm) | Load (kg) | IMEP (bar) | BMEP (bar) | FMEP (bar) |
| --- | --- | --- | --- | --- |
| 1567.00 | 0.32 | 3.78 | 0.11 | 3.67 |
| 1538.00 | 2.63 | 3.72 | 0.91 | 2.81 |
| 1527.00 | 4.54 | 3.75 | 1.57 | 2.19 |
| 1520.00 | 6.24 | 3.73 | 2.15 | 1.58 |
| 1518.00 | 7.05 | 3.21 | 2.43 | 0.78 |

Air &  Fuel Flow

| Speed (rpm) | Load (kg) | Air (mmWC) | Fuel (cc/min) |
| --- | --- | --- | --- |
| 1567.00 | 0.32 | 100.42 | 9.00 |
| 1538.00 | 2.63 | 94.83 | 13.00 |
| 1527.00 | 4.54 | 92.34 | 14.00 |
| 1520.00 | 6.24 | 90.40 | 17.00 |
| 1518.00 | 7.05 | 89.39 | 17.00 |

Indicated & Brake Thermal Efficiency

| Speed (rpm) | Load (kg) | IThEff (%) | BThEff (%) |
| --- | --- | --- | --- |
| 1567.00 | 0.32 | 62.49 | 1.80 |
| 1538.00 | 2.63 | 41.74 | 10.19 |
| 1527.00 | 4.54 | 38.83 | 16.21 |
| 1520.00 | 6.24 | 31.65 | 18.26 |
| 1518.00 | 7.05 | 27.21 | 20.58 |

SFC & Fuel Consumption

| Speed (rpm) | Load (kg) | SFC (kg/kWh) | Fuel (kg/h) |
| --- | --- | --- | --- |
| 1567.00 | 0.32 | 4.75 | 0.45 |
| 1538.00 | 2.63 | 0.84 | 0.65 |
| 1527.00 | 4.54 | 0.53 | 0.70 |
| 1520.00 | 6.24 | 0.47 | 0.85 |
| 1518.00 | 7.05 | 0.42 | 0.85 |

TORQUE, Mechanical & Volmertic Efficiency

| Speed (rpm) | Load (Kg) | Torque (Nm) | Mech Eff. (%) | Vol Eff. (%) |
| --- | --- | --- | --- | --- |
| 1567.00 | 0.32 | 0.58 | 2.89 | 89.40 |
| 1538.00 | 2.63 | 4.78 | 24.42 | 88.52 |
| 1527.00 | 4.54 | 8.25 | 41.75 | 87.98 |
| 1520.00 | 6.24 | 11.33 | 57.70 | 87.45 |
| 1518.00 | 7.05 | 12.79 | 75.66 | 87.07 |

HBP,HJW & HGas

| Speed (rpm) | Load (kg) | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- |
| 1567.00 | 0.32 | 1.80 | 37.14 | 26.11 | 34.95 |
| 1538.00 | 2.63 | 10.19 | 34.76 | 28.00 | 27.04 |
| 1527.00 | 4.54 | 16.21 | 36.97 | 30.07 | 16.76 |
| 1520.00 | 6.24 | 18.26 | 34.46 | 28.13 | 19.15 |
| 1518.00 | 7.05 | 20.58 | 39.18 | 30.58 | 9.66 |

Observation Data

| Speed (rpm) | Load (kg) | Comp Ratio | T1 (deg C) | T2 (deg C) | T3 (deg C) | T4 (deg C) | T5 (deg C) | T6 (deg C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1567 | 0.32 | 17.00 | 19.73 | 25.30 | 19.73 | 23.09 | 162.06 | 116.18 |
| 1538 | 2.63 | 17.00 | 19.71 | 27.24 | 19.72 | 25.11 | 223.08 | 177.33 |
| 1527 | 4.54 | 17.00 | 19.87 | 28.49 | 19.87 | 27.21 | 256.36 | 201.92 |
| 1520 | 6.24 | 17.00 | 19.90 | 29.66 | 19.90 | 29.00 | 289.07 | 239.70 |
| 1518 | 7.05 | 17.00 | 20.08 | 31.17 | 20.09 | 31.20 | 313.42 | 258.32 |

Observation Data

| Air (mmWC) | Fuel (cc/min) | WaterFlow Engine (lph) | WaterFlow Cal (lph) |
| --- | --- | --- | --- |
| 100.42 | 9.00 | 300 | 100 |
| 94.83 | 13.00 | 300 | 100 |
| 92.34 | 14.00 | 300 | 100 |
| 90.40 | 17.00 | 300 | 100 |
| 89.39 | 17.00 | 300 | 100 |

Result Data

| Torque (Nm) | BP (kW) | FP (kW) | IP (kW) | BMEP (bar) | IMEP (bar) | BTHE (%) | ITHE (%) | Mech Eff. (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.58 | 0.09 | 3.17 | 3.27 | 0.11 | 3.78 | 1.80 | 62.49 | 2.89 |
| 4.78 | 0.77 | 2.38 | 3.15 | 0.91 | 3.72 | 10.19 | 41.74 | 24.42 |
| 8.25 | 1.32 | 1.84 | 3.16 | 1.57 | 3.75 | 16.21 | 38.83 | 41.75 |
| 11.33 | 1.80 | 1.32 | 3.13 | 2.15 | 3.73 | 18.26 | 31.65 | 57.70 |
| 12.79 | 2.03 | 0.65 | 2.69 | 2.43 | 3.21 | 20.58 | 27.21 | 75.66 |

Result Data

| Air Flow (kg/h) | Fuel Flow (kg/h) | SFC (kg/kWh) | Vol Eff. (%) | A/F Ratio | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32.63 | 0.45 | 4.75 | 89.40 | 72.81 | 1.80 | 37.14 | 26.11 | 34.95 |
| 31.71 | 0.65 | 0.84 | 88.52 | 48.99 | 10.19 | 34.76 | 28.00 | 27.04 |
| 31.29 | 0.70 | 0.53 | 87.98 | 44.89 | 16.21 | 36.97 | 30.07 | 16.76 |
| 30.96 | 0.85 | 0.47 | 87.45 | 36.57 | 18.26 | 34.46 | 28.13 | 19.15 |
| 30.79 | 0.85 | 0.42 | 87.07 | 36.37 | 20.58 | 39.18 | 30.58 | 9.66 |