ICEngineSoft Test Report

Report Date : Thu, Feb 07, 2019 , Time :  1:49:17 PM

Organization : Organisation, Operator : Operator

Data File : T1 CR 14.xlsx, Last Modified Date: Thu, Feb 07, 2019,

Config. File : VCR Diesel Engine.xls

Engine Details :

ICEngine set up under test is Research Diesel having power 3.50 kW @ 1500 rpm which is 1 Cylinder, Four stroke , Constant Speed, Water Cooled, Diesel Engine, with Cylinder Bore 87.50(mm), Stroke Length 110.00(mm), Connecting Rod length 234.00(mm), Compression Ratio 14.00, Swept volume 661.45 (cc)

Combustion Parameters :

Specific Gas Const (kJ/kgK) : 1.00, Air Density (kg/m^3) : 1.17, Adiabatic Index : 1.41, Polytrophic Index : 0.81, Number Of Cycles : 10, Cylinder Pressure Referance :  2, Smoothing 2, TDC Reference : 0

Performance Parameters :

Orifice Diameter (mm) : 20.00, Orifice Coeff. Of Discharge : 0.60, Dynamometer Arm Legnth (mm) : 185, Fuel Pipe dia (mm) : 12.40, Ambient Temp. (Deg C) : 27, Pulses Per revolution : 360, Fuel Type : Diesel, Fuel Density (Kg/m^3) : 830, Calorific Value Of Fuel (kj/kg) : 42000

IP, BP & FP

| Speed (rpm) | Load (kg) | IP (kW) | BP (kW) | FP (kW) |
| --- | --- | --- | --- | --- |
| 1565.00 | 0.18 | 11.77 | 0.05 | 11.71 |
| 1545.00 | 2.19 | 13.12 | 0.64 | 12.47 |
| 1537.00 | 4.18 | 13.59 | 1.22 | 12.37 |
| 1514.00 | 6.12 | 14.20 | 1.76 | 12.44 |
| 1520.00 | 8.23 | 14.91 | 2.38 | 12.53 |
| 1479.00 | 10.14 | 14.64 | 2.85 | 11.79 |

IMEP,BMEP & FMEP

| Speed (rpm) | Load (kg) | IMEP (bar) | BMEP (bar) | FMEP (bar) |
| --- | --- | --- | --- | --- |
| 1565.00 | 0.18 | 13.64 | 0.06 | 13.58 |
| 1545.00 | 2.19 | 15.40 | 0.76 | 14.65 |
| 1537.00 | 4.18 | 16.05 | 1.44 | 14.60 |
| 1514.00 | 6.12 | 17.01 | 2.11 | 14.90 |
| 1520.00 | 8.23 | 17.79 | 2.84 | 14.96 |
| 1479.00 | 10.14 | 17.95 | 3.50 | 14.46 |

Air &  Fuel Flow

| Speed (rpm) | Load (kg) | Air (mmWC) | Fuel (cc/min) |
| --- | --- | --- | --- |
| 1565.00 | 0.18 | 88.68 | 10.00 |
| 1545.00 | 2.19 | 82.02 | 12.00 |
| 1537.00 | 4.18 | 79.39 | 14.00 |
| 1514.00 | 6.12 | 77.71 | 15.00 |
| 1520.00 | 8.23 | 77.36 | 17.00 |
| 1479.00 | 10.14 | 73.42 | 20.00 |

Indicated & Brake Thermal Efficiency

| Speed (rpm) | Load (kg) | IThEff (%) | BThEff (%) |
| --- | --- | --- | --- |
| 1565.00 | 0.18 | 202.51 | 0.91 |
| 1545.00 | 2.19 | 188.13 | 9.24 |
| 1537.00 | 4.18 | 167.13 | 15.01 |
| 1514.00 | 6.12 | 162.91 | 20.22 |
| 1520.00 | 8.23 | 150.94 | 24.07 |
| 1479.00 | 10.14 | 125.97 | 24.53 |

SFC & Fuel Consumption

| Speed (rpm) | Load (kg) | SFC (kg/kWh) | Fuel (kg/h) |
| --- | --- | --- | --- |
| 1565.00 | 0.18 | 9.47 | 0.50 |
| 1545.00 | 2.19 | 0.93 | 0.60 |
| 1537.00 | 4.18 | 0.57 | 0.70 |
| 1514.00 | 6.12 | 0.42 | 0.75 |
| 1520.00 | 8.23 | 0.36 | 0.85 |
| 1479.00 | 10.14 | 0.35 | 1.00 |

TORQUE, Mechanical & Volmertic Efficiency

| Speed (rpm) | Load (Kg) | Torque (Nm) | Mech Eff. (%) | Vol Eff. (%) |
| --- | --- | --- | --- | --- |
| 1565.00 | 0.18 | 0.32 | 0.45 | 84.12 |
| 1545.00 | 2.19 | 3.98 | 4.91 | 81.95 |
| 1537.00 | 4.18 | 7.59 | 8.98 | 81.04 |
| 1514.00 | 6.12 | 11.11 | 12.41 | 81.40 |
| 1520.00 | 8.23 | 14.93 | 15.95 | 80.89 |
| 1479.00 | 10.14 | 18.41 | 19.48 | 80.99 |

HBP,HJW & HGas

| Speed (rpm) | Load (kg) | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- |
| 1565.00 | 0.18 | 0.91 | 12.27 | 24.70 | 62.12 |
| 1545.00 | 2.19 | 9.24 | 10.18 | 27.64 | 52.94 |
| 1537.00 | 4.18 | 15.01 | 8.73 | 26.65 | 49.61 |
| 1514.00 | 6.12 | 20.22 | 8.14 | 28.12 | 43.52 |
| 1520.00 | 8.23 | 24.07 | 7.22 | 28.56 | 40.15 |
| 1479.00 | 10.14 | 24.53 | 6.06 | 27.06 | 42.35 |

Observation Data

| Speed (rpm) | Load (kg) | Comp Ratio | T1 (deg C) | T2 (deg C) | T3 (deg C) | T4 (deg C) | T5 (deg C) | T6 (deg C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1565 | 0.18 | 14.00 | 21.64 | 24.70 | 21.63 | 25.65 | 165.17 | 140.24 |
| 1545 | 2.19 | 14.00 | 21.65 | 24.71 | 21.65 | 26.21 | 219.13 | 183.46 |
| 1537 | 4.18 | 14.00 | 21.66 | 24.72 | 21.66 | 27.34 | 245.88 | 205.25 |
| 1514 | 6.12 | 14.00 | 21.66 | 24.71 | 21.66 | 28.49 | 276.57 | 228.71 |
| 1520 | 8.23 | 14.00 | 21.67 | 24.73 | 21.67 | 30.27 | 313.94 | 256.90 |
| 1479 | 10.14 | 14.00 | 21.69 | 24.72 | 21.67 | 32.20 | 353.36 | 288.03 |

Observation Data

| Air (mmWC) | Fuel (cc/min) | WaterFlow Engine (lph) | WaterFlow Cal (lph) |
| --- | --- | --- | --- |
| 88.68 | 10.00 | 200 | 100 |
| 82.02 | 12.00 | 200 | 100 |
| 79.39 | 14.00 | 200 | 100 |
| 77.71 | 15.00 | 200 | 100 |
| 77.36 | 17.00 | 200 | 100 |
| 73.42 | 20.00 | 200 | 100 |

Result Data

| Torque (Nm) | BP (kW) | FP (kW) | IP (kW) | BMEP (bar) | IMEP (bar) | BTHE (%) | ITHE (%) | Mech Eff. (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.32 | 0.05 | 11.71 | 11.77 | 0.06 | 13.64 | 0.91 | 202.51 | 0.45 |
| 3.98 | 0.64 | 12.47 | 13.12 | 0.76 | 15.40 | 9.24 | 188.13 | 4.91 |
| 7.59 | 1.22 | 12.37 | 13.59 | 1.44 | 16.05 | 15.01 | 167.13 | 8.98 |
| 11.11 | 1.76 | 12.44 | 14.20 | 2.11 | 17.01 | 20.22 | 162.91 | 12.41 |
| 14.93 | 2.38 | 12.53 | 14.91 | 2.84 | 17.79 | 24.07 | 150.94 | 15.95 |
| 18.41 | 2.85 | 11.79 | 14.64 | 3.50 | 17.95 | 24.53 | 125.97 | 19.48 |

Result Data

| Air Flow (kg/h) | Fuel Flow (kg/h) | SFC (kg/kWh) | Vol Eff. (%) | A/F Ratio | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30.67 | 0.50 | 9.47 | 84.12 | 61.58 | 0.91 | 12.27 | 24.70 | 62.12 |
| 29.49 | 0.60 | 0.93 | 81.95 | 49.35 | 9.24 | 10.18 | 27.64 | 52.94 |
| 29.02 | 0.70 | 0.57 | 81.04 | 41.62 | 15.01 | 8.73 | 26.65 | 49.61 |
| 28.71 | 0.75 | 0.42 | 81.40 | 38.43 | 20.22 | 8.14 | 28.12 | 43.52 |
| 28.64 | 0.85 | 0.36 | 80.89 | 33.83 | 24.07 | 7.22 | 28.56 | 40.15 |
| 27.90 | 1.00 | 0.35 | 80.99 | 28.02 | 24.53 | 6.06 | 27.06 | 42.35 |