ICEngineSoft Test Report

Report Date : Wed, Feb 01, 2017 , Time :  2:54:49 PM

Organization : Organisation, Operator : Operator

Data File : T10 CR17.xlsx, Last Modified Date: Wed, Feb 01, 2017,

Config. File : VCR Diesel Engine.xls

Engine Details :

ICEngine set up under test is Research Diesel having power 3.50 kW @ 1500 rpm which is 1 Cylinder, Four stroke , Constant Speed, Water Cooled, Diesel Engine, with Cylinder Bore 87.50(mm), Stroke Length 110.00(mm), Connecting Rod length 234.00(mm), Compression Ratio 17.00, Swept volume 661.45 (cc)

Combustion Parameters :

Specific Gas Const (kJ/kgK) : 1.00, Air Density (kg/m^3) : 1.17, Adiabatic Index : 1.41, Polytrophic Index : -0.00, Number Of Cycles : 10, Cylinder Pressure Referance :  1, Smoothing 2, TDC Reference : 0

Performance Parameters :

Orifice Diameter (mm) : 20.00, Orifice Coeff. Of Discharge : 0.60, Dynamometer Arm Legnth (mm) : 185, Fuel Pipe dia (mm) : 12.40, Ambient Temp. (Deg C) : 27, Pulses Per revolution : 360, Fuel Type : Diesel, Fuel Density (Kg/m^3) : 830, Calorific Value Of Fuel (kj/kg) : 42000

IP, BP & FP

| Speed (rpm) | Load (kg) | IP (kW) | BP (kW) | FP (kW) |
| --- | --- | --- | --- | --- |
| 1568.00 | 0.20 | 4.51 | 0.06 | 4.45 |
| 1542.00 | 2.21 | 4.45 | 0.65 | 3.81 |
| 1535.00 | 4.11 | 4.60 | 1.20 | 3.40 |
| 1524.00 | 6.32 | 4.36 | 1.83 | 2.54 |
| 1514.00 | 8.23 | 4.22 | 2.37 | 1.85 |

IMEP,BMEP & FMEP

| Speed (rpm) | Load (kg) | IMEP (bar) | BMEP (bar) | FMEP (bar) |
| --- | --- | --- | --- | --- |
| 1568.00 | 0.20 | 5.21 | 0.07 | 5.15 |
| 1542.00 | 2.21 | 5.24 | 0.76 | 4.48 |
| 1535.00 | 4.11 | 5.43 | 1.42 | 4.01 |
| 1524.00 | 6.32 | 5.20 | 2.18 | 3.02 |
| 1514.00 | 8.23 | 5.05 | 2.84 | 2.22 |

Air &  Fuel Flow

| Speed (rpm) | Load (kg) | Air (mmWC) | Fuel (cc/min) |
| --- | --- | --- | --- |
| 1568.00 | 0.20 | 86.70 | 9.00 |
| 1542.00 | 2.21 | 82.74 | 13.00 |
| 1535.00 | 4.11 | 81.48 | 14.00 |
| 1524.00 | 6.32 | 79.73 | 15.00 |
| 1514.00 | 8.23 | 77.06 | 18.00 |

Indicated & Brake Thermal Efficiency

| Speed (rpm) | Load (kg) | IThEff (%) | BThEff (%) |
| --- | --- | --- | --- |
| 1568.00 | 0.20 | 86.17 | 1.13 |
| 1542.00 | 2.21 | 58.97 | 8.57 |
| 1535.00 | 4.11 | 56.51 | 14.76 |
| 1524.00 | 6.32 | 50.08 | 20.99 |
| 1514.00 | 8.23 | 40.33 | 22.65 |

SFC & Fuel Consumption

| Speed (rpm) | Load (kg) | SFC (kg/kWh) | Fuel (kg/h) |
| --- | --- | --- | --- |
| 1568.00 | 0.20 | 7.60 | 0.45 |
| 1542.00 | 2.21 | 1.00 | 0.65 |
| 1535.00 | 4.11 | 0.58 | 0.70 |
| 1524.00 | 6.32 | 0.41 | 0.75 |
| 1514.00 | 8.23 | 0.38 | 0.90 |

TORQUE, Mechanical & Volmertic Efficiency

| Speed (rpm) | Load (Kg) | Torque (Nm) | Mech Eff. (%) | Vol Eff. (%) |
| --- | --- | --- | --- | --- |
| 1568.00 | 0.20 | 0.36 | 1.31 | 83.02 |
| 1542.00 | 2.21 | 4.01 | 14.54 | 82.47 |
| 1535.00 | 4.11 | 7.47 | 26.12 | 82.21 |
| 1524.00 | 6.32 | 11.46 | 41.91 | 81.91 |
| 1514.00 | 8.23 | 14.94 | 56.17 | 81.06 |

HBP,HJW & HGas

| Speed (rpm) | Load (kg) | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- |
| 1568.00 | 0.20 | 1.13 | 44.60 | 25.36 | 28.91 |
| 1542.00 | 2.21 | 8.57 | 34.69 | 24.25 | 32.49 |
| 1535.00 | 4.11 | 14.76 | 35.38 | 26.55 | 23.31 |
| 1524.00 | 6.32 | 20.99 | 36.74 | 28.71 | 13.56 |
| 1514.00 | 8.23 | 22.65 | 32.71 | 27.82 | 16.82 |

Observation Data

| Speed (rpm) | Load (kg) | Comp Ratio | T1 (deg C) | T2 (deg C) | T3 (deg C) | T4 (deg C) | T5 (deg C) | T6 (deg C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1568 | 0.20 | 17.00 | 25.02 | 31.70 | 25.02 | 27.89 | 156.31 | 131.13 |
| 1542 | 2.21 | 17.00 | 25.20 | 32.71 | 25.21 | 29.84 | 208.55 | 173.23 |
| 1535 | 4.11 | 17.00 | 25.30 | 33.55 | 25.31 | 31.84 | 242.25 | 201.74 |
| 1524 | 6.32 | 17.00 | 25.27 | 34.45 | 25.26 | 34.68 | 278.71 | 232.41 |
| 1514 | 8.23 | 17.00 | 25.36 | 35.17 | 25.37 | 37.34 | 323.02 | 265.80 |

Observation Data

| Air (mmWC) | Fuel (cc/min) | WaterFlow Engine (lph) | WaterFlow Cal (lph) |
| --- | --- | --- | --- |
| 86.70 | 9.00 | 300 | 100 |
| 82.74 | 13.00 | 300 | 100 |
| 81.48 | 14.00 | 300 | 100 |
| 79.73 | 15.00 | 300 | 100 |
| 77.06 | 18.00 | 300 | 100 |

Result Data

| Torque (Nm) | BP (kW) | FP (kW) | IP (kW) | BMEP (bar) | IMEP (bar) | BTHE (%) | ITHE (%) | Mech Eff. (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.36 | 0.06 | 4.45 | 4.51 | 0.07 | 5.21 | 1.13 | 86.17 | 1.31 |
| 4.01 | 0.65 | 3.81 | 4.45 | 0.76 | 5.24 | 8.57 | 58.97 | 14.54 |
| 7.47 | 1.20 | 3.40 | 4.60 | 1.42 | 5.43 | 14.76 | 56.51 | 26.12 |
| 11.46 | 1.83 | 2.54 | 4.36 | 2.18 | 5.20 | 20.99 | 50.08 | 41.91 |
| 14.94 | 2.37 | 1.85 | 4.22 | 2.84 | 5.05 | 22.65 | 40.33 | 56.17 |

Result Data

| Air Flow (kg/h) | Fuel Flow (kg/h) | SFC (kg/kWh) | Vol Eff. (%) | A/F Ratio | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30.32 | 0.45 | 7.60 | 83.02 | 67.66 | 1.13 | 44.60 | 25.36 | 28.91 |
| 29.62 | 0.65 | 1.00 | 82.47 | 45.76 | 8.57 | 34.69 | 24.25 | 32.49 |
| 29.40 | 0.70 | 0.58 | 82.21 | 42.16 | 14.76 | 35.38 | 26.55 | 23.31 |
| 29.08 | 0.75 | 0.41 | 81.91 | 38.93 | 20.99 | 36.74 | 28.71 | 13.56 |
| 28.59 | 0.90 | 0.38 | 81.06 | 31.89 | 22.65 | 32.71 | 27.82 | 16.82 |