ICEngineSoft Test Report

Report Date : Wed, Feb 01, 2017 , Time :  4:13:36 PM

Organization : Organisation, Operator : Operator

Data File : T12CR 18.xlsx, Last Modified Date: Wed, Feb 01, 2017,

Config. File : VCR Diesel Engine.xls

Engine Details :

ICEngine set up under test is Research Diesel having power 3.50 kW @ 1500 rpm which is 1 Cylinder, Four stroke , Constant Speed, Water Cooled, Diesel Engine, with Cylinder Bore 87.50(mm), Stroke Length 110.00(mm), Connecting Rod length 234.00(mm), Compression Ratio 18.00, Swept volume 661.45 (cc)

Combustion Parameters :

Specific Gas Const (kJ/kgK) : 1.00, Air Density (kg/m^3) : 1.17, Adiabatic Index : 1.41, Polytrophic Index : -0.00, Number Of Cycles : 10, Cylinder Pressure Referance :  1, Smoothing 2, TDC Reference : 0

Performance Parameters :

Orifice Diameter (mm) : 20.00, Orifice Coeff. Of Discharge : 0.60, Dynamometer Arm Legnth (mm) : 185, Fuel Pipe dia (mm) : 12.40, Ambient Temp. (Deg C) : 27, Pulses Per revolution : 360, Fuel Type : Diesel, Fuel Density (Kg/m^3) : 830, Calorific Value Of Fuel (kj/kg) : 42000

IP, BP & FP

| Speed (rpm) | Load (kg) | IP (kW) | BP (kW) | FP (kW) |
| --- | --- | --- | --- | --- |
| 1566.00 | 0.21 | 5.37 | 0.06 | 5.31 |
| 1544.00 | 2.32 | 5.54 | 0.68 | 4.86 |
| 1531.00 | 4.33 | 5.29 | 1.26 | 4.03 |
| 1524.00 | 6.13 | 5.37 | 1.78 | 3.60 |
| 1515.00 | 8.24 | 5.27 | 2.37 | 2.89 |

IMEP,BMEP & FMEP

| Speed (rpm) | Load (kg) | IMEP (bar) | BMEP (bar) | FMEP (bar) |
| --- | --- | --- | --- | --- |
| 1566.00 | 0.21 | 6.22 | 0.07 | 6.15 |
| 1544.00 | 2.32 | 6.51 | 0.80 | 5.71 |
| 1531.00 | 4.33 | 6.27 | 1.49 | 4.78 |
| 1524.00 | 6.13 | 6.40 | 2.11 | 4.28 |
| 1515.00 | 8.24 | 6.31 | 2.84 | 3.47 |

Air &  Fuel Flow

| Speed (rpm) | Load (kg) | Air (mmWC) | Fuel (cc/min) |
| --- | --- | --- | --- |
| 1566.00 | 0.21 | 85.14 | 7.00 |
| 1544.00 | 2.32 | 82.12 | 11.00 |
| 1531.00 | 4.33 | 79.55 | 13.00 |
| 1524.00 | 6.13 | 78.02 | 15.00 |
| 1515.00 | 8.24 | 77.22 | 17.00 |

Indicated & Brake Thermal Efficiency

| Speed (rpm) | Load (kg) | IThEff (%) | BThEff (%) |
| --- | --- | --- | --- |
| 1566.00 | 0.21 | 132.03 | 1.55 |
| 1544.00 | 2.32 | 86.69 | 10.65 |
| 1531.00 | 4.33 | 70.07 | 16.69 |
| 1524.00 | 6.13 | 61.65 | 20.38 |
| 1515.00 | 8.24 | 53.32 | 24.02 |

SFC & Fuel Consumption

| Speed (rpm) | Load (kg) | SFC (kg/kWh) | Fuel (kg/h) |
| --- | --- | --- | --- |
| 1566.00 | 0.21 | 5.52 | 0.35 |
| 1544.00 | 2.32 | 0.80 | 0.55 |
| 1531.00 | 4.33 | 0.51 | 0.65 |
| 1524.00 | 6.13 | 0.42 | 0.75 |
| 1515.00 | 8.24 | 0.36 | 0.85 |

TORQUE, Mechanical & Volmertic Efficiency

| Speed (rpm) | Load (Kg) | Torque (Nm) | Mech Eff. (%) | Vol Eff. (%) |
| --- | --- | --- | --- | --- |
| 1566.00 | 0.21 | 0.38 | 1.18 | 82.38 |
| 1544.00 | 2.32 | 4.21 | 12.28 | 82.06 |
| 1531.00 | 4.33 | 7.86 | 23.81 | 81.45 |
| 1524.00 | 6.13 | 11.13 | 33.06 | 81.03 |
| 1515.00 | 8.24 | 14.95 | 45.04 | 81.09 |

HBP,HJW & HGas

| Speed (rpm) | Load (kg) | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- |
| 1566.00 | 0.21 | 1.55 | 58.35 | 32.41 | 7.69 |
| 1544.00 | 2.32 | 10.65 | 38.23 | 27.26 | 23.87 |
| 1531.00 | 4.33 | 16.69 | 35.38 | 26.79 | 21.14 |
| 1524.00 | 6.13 | 20.38 | 33.53 | 26.74 | 19.35 |
| 1515.00 | 8.24 | 24.02 | 31.45 | 26.98 | 17.55 |

Observation Data

| Speed (rpm) | Load (kg) | Comp Ratio | T1 (deg C) | T2 (deg C) | T3 (deg C) | T4 (deg C) | T5 (deg C) | T6 (deg C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1566 | 0.21 | 18.00 | 26.32 | 33.13 | 26.32 | 29.61 | 157.08 | 138.83 |
| 1544 | 2.32 | 18.00 | 26.39 | 33.39 | 26.39 | 30.36 | 200.84 | 170.64 |
| 1531 | 4.33 | 18.00 | 26.40 | 34.06 | 26.40 | 31.47 | 231.42 | 194.85 |
| 1524 | 6.13 | 18.00 | 26.41 | 34.78 | 26.41 | 32.81 | 263.88 | 220.70 |
| 1515 | 8.24 | 18.00 | 26.48 | 35.39 | 26.48 | 34.29 | 298.37 | 248.18 |

Observation Data

| Air (mmWC) | Fuel (cc/min) | WaterFlow Engine (lph) | WaterFlow Cal (lph) |
| --- | --- | --- | --- |
| 85.14 | 7.00 | 300 | 100 |
| 82.12 | 11.00 | 300 | 100 |
| 79.55 | 13.00 | 300 | 100 |
| 78.02 | 15.00 | 300 | 100 |
| 77.22 | 17.00 | 300 | 100 |

Result Data

| Torque (Nm) | BP (kW) | FP (kW) | IP (kW) | BMEP (bar) | IMEP (bar) | BTHE (%) | ITHE (%) | Mech Eff. (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.38 | 0.06 | 5.31 | 5.37 | 0.07 | 6.22 | 1.55 | 132.03 | 1.18 |
| 4.21 | 0.68 | 4.86 | 5.54 | 0.80 | 6.51 | 10.65 | 86.69 | 12.28 |
| 7.86 | 1.26 | 4.03 | 5.29 | 1.49 | 6.27 | 16.69 | 70.07 | 23.81 |
| 11.13 | 1.78 | 3.60 | 5.37 | 2.11 | 6.40 | 20.38 | 61.65 | 33.06 |
| 14.95 | 2.37 | 2.89 | 5.27 | 2.84 | 6.31 | 24.02 | 53.32 | 45.04 |

Result Data

| Air Flow (kg/h) | Fuel Flow (kg/h) | SFC (kg/kWh) | Vol Eff. (%) | A/F Ratio | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30.05 | 0.35 | 5.52 | 82.38 | 86.20 | 1.55 | 58.35 | 32.41 | 7.69 |
| 29.51 | 0.55 | 0.80 | 82.06 | 53.87 | 10.65 | 38.23 | 27.26 | 23.87 |
| 29.05 | 0.65 | 0.51 | 81.45 | 44.87 | 16.69 | 35.38 | 26.79 | 21.14 |
| 28.77 | 0.75 | 0.42 | 81.03 | 38.51 | 20.38 | 33.53 | 26.74 | 19.35 |
| 28.62 | 0.85 | 0.36 | 81.09 | 33.80 | 24.02 | 31.45 | 26.98 | 17.55 |