ICEngineSoft Test Report

Report Date : Fri, Jan 27, 2017 , Time :  1:48:50 PM

Organization : Organisation, Operator : Operator

Data File : T9NEW.COMPUTER.xlsx, Last Modified Date: Fri, Jan 27, 2017,

Config. File : VCR Diesel Engine.xls

Engine Details :

ICEngine set up under test is Research Diesel having power 3.50 kW @ 1500 rpm which is 1 Cylinder, Four stroke , Constant Speed, Water Cooled, Diesel Engine, with Cylinder Bore 87.50(mm), Stroke Length 110.00(mm), Connecting Rod length 234.00(mm), Compression Ratio 17.00, Swept volume 661.45 (cc)

Combustion Parameters :

Specific Gas Const (kJ/kgK) : 1.00, Air Density (kg/m^3) : 1.17, Adiabatic Index : 1.41, Polytrophic Index : -0.00, Number Of Cycles : 10, Cylinder Pressure Referance :  2, Smoothing 2, TDC Reference : 0

Performance Parameters :

Orifice Diameter (mm) : 20.00, Orifice Coeff. Of Discharge : 0.60, Dynamometer Arm Legnth (mm) : 185, Fuel Pipe dia (mm) : 12.40, Ambient Temp. (Deg C) : 27, Pulses Per revolution : 360, Fuel Type : Diesel, Fuel Density (Kg/m^3) : 830, Calorific Value Of Fuel (kj/kg) : 42000

IP, BP & FP

| Speed (rpm) | Load (kg) | IP (kW) | BP (kW) | FP (kW) |
| --- | --- | --- | --- | --- |
| 1545.00 | 2.41 | 4.42 | 0.71 | 3.71 |
| 1521.00 | 4.11 | 4.80 | 1.19 | 3.61 |
| 1523.00 | 6.11 | 4.31 | 1.77 | 2.54 |
| 1518.00 | 8.07 | 5.12 | 2.33 | 2.79 |

IMEP,BMEP & FMEP

| Speed (rpm) | Load (kg) | IMEP (bar) | BMEP (bar) | FMEP (bar) |
| --- | --- | --- | --- | --- |
| 1545.00 | 2.41 | 5.19 | 0.83 | 4.36 |
| 1521.00 | 4.11 | 5.73 | 1.42 | 4.31 |
| 1523.00 | 6.11 | 5.13 | 2.10 | 3.03 |
| 1518.00 | 8.07 | 6.11 | 2.78 | 3.33 |

Air &  Fuel Flow

| Speed (rpm) | Load (kg) | Air (mmWC) | Fuel (cc/min) |
| --- | --- | --- | --- |
| 1545.00 | 2.41 | 92.26 | 11.00 |
| 1521.00 | 4.11 | 89.24 | 12.00 |
| 1523.00 | 6.11 | 88.63 | 15.00 |
| 1518.00 | 8.07 | 85.45 | 16.00 |

Indicated & Brake Thermal Efficiency

| Speed (rpm) | Load (kg) | IThEff (%) | BThEff (%) |
| --- | --- | --- | --- |
| 1545.00 | 2.41 | 69.16 | 11.08 |
| 1521.00 | 4.11 | 68.88 | 17.05 |
| 1523.00 | 6.11 | 49.46 | 20.28 |
| 1518.00 | 8.07 | 55.03 | 25.05 |

SFC & Fuel Consumption

| Speed (rpm) | Load (kg) | SFC (kg/kWh) | Fuel (kg/h) |
| --- | --- | --- | --- |
| 1545.00 | 2.41 | 0.77 | 0.55 |
| 1521.00 | 4.11 | 0.50 | 0.60 |
| 1523.00 | 6.11 | 0.42 | 0.75 |
| 1518.00 | 8.07 | 0.34 | 0.80 |

TORQUE, Mechanical & Volmertic Efficiency

| Speed (rpm) | Load (Kg) | Torque (Nm) | Mech Eff. (%) | Vol Eff. (%) |
| --- | --- | --- | --- | --- |
| 1545.00 | 2.41 | 4.38 | 16.02 | 86.91 |
| 1521.00 | 4.11 | 7.46 | 24.75 | 86.83 |
| 1523.00 | 6.11 | 11.08 | 41.00 | 86.42 |
| 1518.00 | 8.07 | 14.65 | 45.52 | 85.14 |

HBP,HJW & HGas

| Speed (rpm) | Load (kg) | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- |
| 1545.00 | 2.41 | 11.08 | 61.68 | 32.44 | 0.00 |
| 1521.00 | 4.11 | 17.05 | 57.08 | 32.57 | 0.00 |
| 1523.00 | 6.11 | 20.28 | 53.13 | 29.59 | 0.00 |
| 1518.00 | 8.07 | 25.05 | 60.62 | 31.96 | 0.00 |

Observation Data

| Speed (rpm) | Load (kg) | Comp Ratio | T1 (deg C) | T2 (deg C) | T3 (deg C) | T4 (deg C) | T5 (deg C) | T6 (deg C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1545 | 2.41 | 17.00 | 27.08 | 38.38 | 27.07 | 39.20 | 222.39 | 193.32 |
| 1521 | 4.11 | 17.00 | 27.22 | 38.63 | 27.23 | 39.62 | 244.22 | 209.53 |
| 1523 | 6.11 | 17.00 | 27.28 | 40.56 | 27.28 | 41.88 | 273.31 | 232.38 |
| 1518 | 8.07 | 17.00 | 27.34 | 43.49 | 27.35 | 45.30 | 315.42 | 264.89 |

Observation Data

| Air (mmWC) | Fuel (cc/min) | WaterFlow Engine (lph) | WaterFlow Cal (lph) |
| --- | --- | --- | --- |
| 92.26 | 11.00 | 300 | 100 |
| 89.24 | 12.00 | 300 | 100 |
| 88.63 | 15.00 | 300 | 100 |
| 85.45 | 16.00 | 300 | 100 |

Result Data

| Torque (Nm) | BP (kW) | FP (kW) | IP (kW) | BMEP (bar) | IMEP (bar) | BTHE (%) | ITHE (%) | Mech Eff. (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.38 | 0.71 | 3.71 | 4.42 | 0.83 | 5.19 | 11.08 | 69.16 | 16.02 |
| 7.46 | 1.19 | 3.61 | 4.80 | 1.42 | 5.73 | 17.05 | 68.88 | 24.75 |
| 11.08 | 1.77 | 2.54 | 4.31 | 2.10 | 5.13 | 20.28 | 49.46 | 41.00 |
| 14.65 | 2.33 | 2.79 | 5.12 | 2.78 | 6.11 | 25.05 | 55.03 | 45.52 |

Result Data

| Air Flow (kg/h) | Fuel Flow (kg/h) | SFC (kg/kWh) | Vol Eff. (%) | A/F Ratio | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31.28 | 0.55 | 0.77 | 86.91 | 57.10 | 11.08 | 61.68 | 32.44 | 0.00 |
| 30.76 | 0.60 | 0.50 | 86.83 | 51.48 | 17.05 | 57.08 | 32.57 | 0.00 |
| 30.66 | 0.75 | 0.42 | 86.42 | 41.04 | 20.28 | 53.13 | 29.59 | 0.00 |
| 30.10 | 0.80 | 0.34 | 85.14 | 37.78 | 25.05 | 60.62 | 31.96 | 0.00 |