ICEngineSoft Test Report

Report Date : Tue, Jan 31, 2017 , Time :  10:21:19 AM

Organization : Organisation, Operator : Operator

Data File : T11.CR 15.xlsx, Last Modified Date: Tue, Jan 31, 2017,

Config. File : VCR Diesel Engine.xls

Engine Details :

ICEngine set up under test is Research Diesel having power 3.50 kW @ 1500 rpm which is 1 Cylinder, Four stroke , Constant Speed, Water Cooled, Diesel Engine, with Cylinder Bore 87.50(mm), Stroke Length 110.00(mm), Connecting Rod length 234.00(mm), Compression Ratio 15.00, Swept volume 661.45 (cc)

Combustion Parameters :

Specific Gas Const (kJ/kgK) : 1.00, Air Density (kg/m^3) : 1.17, Adiabatic Index : 1.41, Polytrophic Index : -0.00, Number Of Cycles : 10, Cylinder Pressure Referance :  1, Smoothing 2, TDC Reference : 0

Performance Parameters :

Orifice Diameter (mm) : 20.00, Orifice Coeff. Of Discharge : 0.60, Dynamometer Arm Legnth (mm) : 185, Fuel Pipe dia (mm) : 12.40, Ambient Temp. (Deg C) : 27, Pulses Per revolution : 360, Fuel Type : Diesel, Fuel Density (Kg/m^3) : 830, Calorific Value Of Fuel (kj/kg) : 42000

IP, BP & FP

| Speed (rpm) | Load (kg) | IP (kW) | BP (kW) | FP (kW) |
| --- | --- | --- | --- | --- |
| 1564.00 | 0.19 | 3.49 | 0.06 | 3.43 |
| 1543.00 | 2.23 | 3.25 | 0.65 | 2.60 |
| 1519.00 | 4.31 | 3.13 | 1.24 | 1.89 |
| 1485.00 | 6.02 | 2.43 | 1.70 | 0.73 |
| 1484.00 | 8.14 | 2.03 | 2.30 | 0.00 |

IMEP,BMEP & FMEP

| Speed (rpm) | Load (kg) | IMEP (bar) | BMEP (bar) | FMEP (bar) |
| --- | --- | --- | --- | --- |
| 1564.00 | 0.19 | 4.04 | 0.07 | 3.98 |
| 1543.00 | 2.23 | 3.83 | 0.77 | 3.06 |
| 1519.00 | 4.31 | 3.74 | 1.48 | 2.26 |
| 1485.00 | 6.02 | 2.97 | 2.08 | 0.89 |
| 1484.00 | 8.14 | 2.48 | 2.81 | -0.33 |

Air &  Fuel Flow

| Speed (rpm) | Load (kg) | Air (mmWC) | Fuel (cc/min) |
| --- | --- | --- | --- |
| 1564.00 | 0.19 | 97.56 | 8.00 |
| 1543.00 | 2.23 | 93.16 | 12.00 |
| 1519.00 | 4.31 | 91.23 | 14.00 |
| 1485.00 | 6.02 | 89.22 | 16.00 |
| 1484.00 | 8.14 | 85.84 | 18.00 |

Indicated & Brake Thermal Efficiency

| Speed (rpm) | Load (kg) | IThEff (%) | BThEff (%) |
| --- | --- | --- | --- |
| 1564.00 | 0.19 | 75.02 | 1.22 |
| 1543.00 | 2.23 | 46.66 | 9.37 |
| 1519.00 | 4.31 | 38.52 | 15.28 |
| 1485.00 | 6.02 | 26.15 | 18.28 |
| 1484.00 | 8.14 | 19.42 | 21.96 |

SFC & Fuel Consumption

| Speed (rpm) | Load (kg) | SFC (kg/kWh) | Fuel (kg/h) |
| --- | --- | --- | --- |
| 1564.00 | 0.19 | 7.03 | 0.40 |
| 1543.00 | 2.23 | 0.92 | 0.60 |
| 1519.00 | 4.31 | 0.56 | 0.70 |
| 1485.00 | 6.02 | 0.47 | 0.80 |
| 1484.00 | 8.14 | 0.39 | 0.90 |

TORQUE, Mechanical & Volmertic Efficiency

| Speed (rpm) | Load (Kg) | Torque (Nm) | Mech Eff. (%) | Vol Eff. (%) |
| --- | --- | --- | --- | --- |
| 1564.00 | 0.19 | 0.35 | 1.63 | 88.29 |
| 1543.00 | 2.23 | 4.04 | 20.07 | 87.45 |
| 1519.00 | 4.31 | 7.81 | 39.66 | 87.91 |
| 1485.00 | 6.02 | 10.93 | 69.89 | 88.92 |
| 1484.00 | 8.14 | 14.78 | 113.11 | 87.28 |

HBP,HJW & HGas

| Speed (rpm) | Load (kg) | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- |
| 1564.00 | 0.19 | 1.22 | 53.77 | 35.64 | 9.37 |
| 1543.00 | 2.23 | 9.37 | 43.83 | 30.91 | 15.89 |
| 1519.00 | 4.31 | 15.28 | 44.29 | 29.99 | 10.44 |
| 1485.00 | 6.02 | 18.28 | 46.80 | 29.52 | 5.40 |
| 1484.00 | 8.14 | 21.96 | 48.48 | 30.09 | 0.00 |

Observation Data

| Speed (rpm) | Load (kg) | Comp Ratio | T1 (deg C) | T2 (deg C) | T3 (deg C) | T4 (deg C) | T5 (deg C) | T6 (deg C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1564 | 0.19 | 15.00 | 23.76 | 30.93 | 23.76 | 27.77 | 179.62 | 158.91 |
| 1543 | 2.23 | 15.00 | 23.83 | 32.59 | 23.84 | 30.68 | 228.87 | 194.89 |
| 1519 | 4.31 | 15.00 | 23.86 | 34.19 | 23.86 | 33.24 | 257.11 | 217.67 |
| 1485 | 6.02 | 15.00 | 23.91 | 36.38 | 23.90 | 35.89 | 287.85 | 242.00 |
| 1484 | 8.14 | 15.00 | 23.95 | 38.48 | 23.95 | 38.29 | 330.88 | 274.83 |

Observation Data

| Air (mmWC) | Fuel (cc/min) | WaterFlow Engine (lph) | WaterFlow Cal (lph) |
| --- | --- | --- | --- |
| 97.56 | 8.00 | 300 | 100 |
| 93.16 | 12.00 | 300 | 100 |
| 91.23 | 14.00 | 300 | 100 |
| 89.22 | 16.00 | 300 | 100 |
| 85.84 | 18.00 | 300 | 100 |

Result Data

| Torque (Nm) | BP (kW) | FP (kW) | IP (kW) | BMEP (bar) | IMEP (bar) | BTHE (%) | ITHE (%) | Mech Eff. (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.35 | 0.06 | 3.43 | 3.49 | 0.07 | 4.04 | 1.22 | 75.02 | 1.63 |
| 4.04 | 0.65 | 2.60 | 3.25 | 0.77 | 3.83 | 9.37 | 46.66 | 20.07 |
| 7.81 | 1.24 | 1.89 | 3.13 | 1.48 | 3.74 | 15.28 | 38.52 | 39.66 |
| 10.93 | 1.70 | 0.73 | 2.43 | 2.08 | 2.97 | 18.28 | 26.15 | 69.89 |
| 14.78 | 2.30 | 0.00 | 2.03 | 2.81 | 2.48 | 21.96 | 19.42 | 113.11 |

Result Data

| Air Flow (kg/h) | Fuel Flow (kg/h) | SFC (kg/kWh) | Vol Eff. (%) | A/F Ratio | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32.17 | 0.40 | 7.03 | 88.29 | 80.74 | 1.22 | 53.77 | 35.64 | 9.37 |
| 31.43 | 0.60 | 0.92 | 87.45 | 52.60 | 9.37 | 43.83 | 30.91 | 15.89 |
| 31.11 | 0.70 | 0.56 | 87.91 | 44.61 | 15.28 | 44.29 | 29.99 | 10.44 |
| 30.76 | 0.80 | 0.47 | 88.92 | 38.61 | 18.28 | 46.80 | 29.52 | 5.40 |
| 30.17 | 0.90 | 0.39 | 87.28 | 33.66 | 21.96 | 48.48 | 30.09 | 0.00 |