ICEngineSoft Test Report

Report Date : Thu, Jan 25, 2018 , Time :  4:27:08 PM

Organization : Organisation, Operator : Operator

Data File : T2 CR16.xlsx, Last Modified Date: Thu, Jan 25, 2018,

Config. File : VCR Diesel Engine.xls

Engine Details :

ICEngine set up under test is Research Diesel having power 3.50 kW @ 1500 rpm which is 1 Cylinder, Four stroke , Constant Speed, Water Cooled, Diesel Engine, with Cylinder Bore 87.50(mm), Stroke Length 110.00(mm), Connecting Rod length 234.00(mm), Compression Ratio 16.00, Swept volume 661.45 (cc)

Combustion Parameters :

Specific Gas Const (kJ/kgK) : 1.00, Air Density (kg/m^3) : 1.17, Adiabatic Index : 1.41, Polytrophic Index : -0.00, Number Of Cycles : 10, Cylinder Pressure Referance :  0, Smoothing 2, TDC Reference : 0

Performance Parameters :

Orifice Diameter (mm) : 20.00, Orifice Coeff. Of Discharge : 0.60, Dynamometer Arm Legnth (mm) : 185, Fuel Pipe dia (mm) : 12.40, Ambient Temp. (Deg C) : 27, Pulses Per revolution : 360, Fuel Type : Diesel, Fuel Density (Kg/m^3) : 830, Calorific Value Of Fuel (kj/kg) : 42000

IP, BP & FP

| Speed (rpm) | Load (kg) | IP (kW) | BP (kW) | FP (kW) |
| --- | --- | --- | --- | --- |
| 1613.00 | 0.21 | 1.39 | 0.06 | 1.32 |
| 1542.00 | 2.26 | 1.46 | 0.66 | 0.79 |
| 1485.00 | 4.02 | 1.52 | 1.14 | 0.38 |
| 1526.00 | 6.04 | 1.64 | 1.75 | 0.00 |
| 1514.00 | 8.15 | 1.75 | 2.35 | 0.00 |

IMEP,BMEP & FMEP

| Speed (rpm) | Load (kg) | IMEP (bar) | BMEP (bar) | FMEP (bar) |
| --- | --- | --- | --- | --- |
| 1613.00 | 0.21 | 1.56 | 0.07 | 1.49 |
| 1542.00 | 2.26 | 1.71 | 0.78 | 0.94 |
| 1485.00 | 4.02 | 1.85 | 1.39 | 0.47 |
| 1526.00 | 6.04 | 1.95 | 2.08 | -0.13 |
| 1514.00 | 8.15 | 2.10 | 2.81 | -0.71 |

Air &  Fuel Flow

| Speed (rpm) | Load (kg) | Air (mmWC) | Fuel (cc/min) |
| --- | --- | --- | --- |
| 1613.00 | 0.21 | 99.96 | 10.00 |
| 1542.00 | 2.26 | 93.87 | 13.00 |
| 1485.00 | 4.02 | 92.12 | 15.00 |
| 1526.00 | 6.04 | 88.81 | 10.00 |
| 1514.00 | 8.15 | 86.87 | 0.00 |

Indicated & Brake Thermal Efficiency

| Speed (rpm) | Load (kg) | IThEff (%) | BThEff (%) |
| --- | --- | --- | --- |
| 1613.00 | 0.21 | 23.90 | 1.09 |
| 1542.00 | 2.26 | 19.29 | 8.76 |
| 1485.00 | 4.02 | 17.42 | 13.03 |
| 1526.00 | 6.04 | 28.29 | 30.13 |
| 1514.00 | 8.15 | 0.00 | 0.00 |

SFC & Fuel Consumption

| Speed (rpm) | Load (kg) | SFC (kg/kWh) | Fuel (kg/h) |
| --- | --- | --- | --- |
| 1613.00 | 0.21 | 7.84 | 0.50 |
| 1542.00 | 2.26 | 0.98 | 0.65 |
| 1485.00 | 4.02 | 0.66 | 0.75 |
| 1526.00 | 6.04 | 0.28 | 0.50 |
| 1514.00 | 8.15 | 0.00 | 0.00 |

TORQUE, Mechanical & Volmertic Efficiency

| Speed (rpm) | Load (Kg) | Torque (Nm) | Mech Eff. (%) | Vol Eff. (%) |
| --- | --- | --- | --- | --- |
| 1613.00 | 0.21 | 0.38 | 4.58 | 86.66 |
| 1542.00 | 2.26 | 4.10 | 45.44 | 87.84 |
| 1485.00 | 4.02 | 7.30 | 74.78 | 90.36 |
| 1526.00 | 6.04 | 10.95 | 106.49 | 86.34 |
| 1514.00 | 8.15 | 14.79 | 133.97 | 86.06 |

HBP,HJW & HGas

| Speed (rpm) | Load (kg) | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- |
| 1613.00 | 0.21 | 1.09 | 27.93 | 28.12 | 42.86 |
| 1542.00 | 2.26 | 8.76 | 358.64 | 28.35 | 0.00 |
| 1485.00 | 4.02 | 13.03 | 201.70 | 27.87 | 0.00 |
| 1526.00 | 6.04 | 30.13 | 500.25 | 47.42 | 0.00 |
| 1514.00 | 8.15 | 0.00 | 0.00 | 0.00 | 100.00 |

Observation Data

| Speed (rpm) | Load (kg) | Comp Ratio | T1 (deg C) | T2 (deg C) | T3 (deg C) | T4 (deg C) | T5 (deg C) | T6 (deg C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1613 | 0.21 | 16.00 | 27.00 | 31.65 | 27.00 | 29.18 | 175.25 | 134.86 |
| 1542 | 2.26 | 16.00 | 26.98 | 104.63 | 27.00 | 31.54 | 226.47 | 183.21 |
| 1485 | 4.02 | 16.00 | 26.97 | 77.36 | 26.98 | 33.94 | 254.66 | 187.41 |
| 1526 | 6.04 | 16.00 | 26.96 | 110.28 | 26.98 | 36.53 | 292.02 | 212.66 |
| 1514 | 8.15 | 16.00 | 26.97 | 102.42 | 26.98 | 40.23 | 331.13 | 243.08 |

Observation Data

| Air (mmWC) | Fuel (cc/min) | WaterFlow Engine (lph) | WaterFlow Cal (lph) |
| --- | --- | --- | --- |
| 99.96 | 10.00 | 300 | 200 |
| 93.87 | 13.00 | 300 | 200 |
| 92.12 | 15.00 | 300 | 200 |
| 88.81 | 10.00 | 300 | 200 |
| 86.87 | 0.00 | 300 | 200 |

Result Data

| Torque (Nm) | BP (kW) | FP (kW) | IP (kW) | BMEP (bar) | IMEP (bar) | BTHE (%) | ITHE (%) | Mech Eff. (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.38 | 0.06 | 1.32 | 1.39 | 0.07 | 1.56 | 1.09 | 23.90 | 4.58 |
| 4.10 | 0.66 | 0.79 | 1.46 | 0.78 | 1.71 | 8.76 | 19.29 | 45.44 |
| 7.30 | 1.14 | 0.38 | 1.52 | 1.39 | 1.85 | 13.03 | 17.42 | 74.78 |
| 10.95 | 1.75 | 0.00 | 1.64 | 2.08 | 1.95 | 30.13 | 28.29 | 106.49 |
| 14.79 | 2.35 | 0.00 | 1.75 | 2.81 | 2.10 | 0.00 | 0.00 | 133.97 |

Result Data

| Air Flow (kg/h) | Fuel Flow (kg/h) | SFC (kg/kWh) | Vol Eff. (%) | A/F Ratio | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32.56 | 0.50 | 7.84 | 86.66 | 65.38 | 1.09 | 27.93 | 28.12 | 42.86 |
| 31.55 | 0.65 | 0.98 | 87.84 | 48.74 | 8.76 | 358.64 | 28.35 | 0.00 |
| 31.26 | 0.75 | 0.66 | 90.36 | 41.84 | 13.03 | 201.70 | 27.87 | 0.00 |
| 30.69 | 0.50 | 0.28 | 86.34 | 61.63 | 30.13 | 500.25 | 47.42 | 0.00 |
| 30.35 | 0.00 | 0.00 | 86.06 | 0.00 | 0.00 | 0.00 | 0.00 | 100.00 |