ICEngineSoft Test Report

Report Date : Wed, Feb 01, 2017 , Time :  3:44:12 PM

Organization : Organisation, Operator : Operator

Data File : T12 CR15.xlsx, Last Modified Date: Wed, Feb 01, 2017,

Config. File : VCR Diesel Engine.xls

Engine Details :

ICEngine set up under test is Research Diesel having power 3.50 kW @ 1500 rpm which is 1 Cylinder, Four stroke , Constant Speed, Water Cooled, Diesel Engine, with Cylinder Bore 87.50(mm), Stroke Length 110.00(mm), Connecting Rod length 234.00(mm), Compression Ratio 15.00, Swept volume 661.45 (cc)

Combustion Parameters :

Specific Gas Const (kJ/kgK) : 1.00, Air Density (kg/m^3) : 1.17, Adiabatic Index : 1.41, Polytrophic Index : -0.00, Number Of Cycles : 10, Cylinder Pressure Referance :  1, Smoothing 2, TDC Reference : 0

Performance Parameters :

Orifice Diameter (mm) : 20.00, Orifice Coeff. Of Discharge : 0.60, Dynamometer Arm Legnth (mm) : 185, Fuel Pipe dia (mm) : 12.40, Ambient Temp. (Deg C) : 27, Pulses Per revolution : 360, Fuel Type : Diesel, Fuel Density (Kg/m^3) : 830, Calorific Value Of Fuel (kj/kg) : 42000

IP, BP & FP

| Speed (rpm) | Load (kg) | IP (kW) | BP (kW) | FP (kW) |
| --- | --- | --- | --- | --- |
| 1523.00 | 0.21 | 4.39 | 0.06 | 4.33 |
| 1540.00 | 2.31 | 4.36 | 0.68 | 3.69 |
| 1551.00 | 4.13 | 4.18 | 1.22 | 2.96 |
| 1519.00 | 6.24 | 4.03 | 1.80 | 2.23 |
| 1509.00 | 8.16 | 3.63 | 2.34 | 1.29 |

IMEP,BMEP & FMEP

| Speed (rpm) | Load (kg) | IMEP (bar) | BMEP (bar) | FMEP (bar) |
| --- | --- | --- | --- | --- |
| 1523.00 | 0.21 | 5.24 | 0.07 | 5.16 |
| 1540.00 | 2.31 | 5.14 | 0.80 | 4.34 |
| 1551.00 | 4.13 | 4.88 | 1.42 | 3.46 |
| 1519.00 | 6.24 | 4.82 | 2.15 | 2.67 |
| 1509.00 | 8.16 | 4.37 | 2.81 | 1.55 |

Air &  Fuel Flow

| Speed (rpm) | Load (kg) | Air (mmWC) | Fuel (cc/min) |
| --- | --- | --- | --- |
| 1523.00 | 0.21 | 84.96 | 9.00 |
| 1540.00 | 2.31 | 81.76 | 13.00 |
| 1551.00 | 4.13 | 80.58 | 13.00 |
| 1519.00 | 6.24 | 78.73 | 14.00 |
| 1509.00 | 8.16 | 77.43 | 18.00 |

Indicated & Brake Thermal Efficiency

| Speed (rpm) | Load (kg) | IThEff (%) | BThEff (%) |
| --- | --- | --- | --- |
| 1523.00 | 0.21 | 84.05 | 1.16 |
| 1540.00 | 2.31 | 57.74 | 8.94 |
| 1551.00 | 4.13 | 55.29 | 16.11 |
| 1519.00 | 6.24 | 49.61 | 22.16 |
| 1509.00 | 8.16 | 34.73 | 22.39 |

SFC & Fuel Consumption

| Speed (rpm) | Load (kg) | SFC (kg/kWh) | Fuel (kg/h) |
| --- | --- | --- | --- |
| 1523.00 | 0.21 | 7.37 | 0.45 |
| 1540.00 | 2.31 | 0.96 | 0.65 |
| 1551.00 | 4.13 | 0.53 | 0.65 |
| 1519.00 | 6.24 | 0.39 | 0.70 |
| 1509.00 | 8.16 | 0.38 | 0.90 |

TORQUE, Mechanical & Volmertic Efficiency

| Speed (rpm) | Load (Kg) | Torque (Nm) | Mech Eff. (%) | Vol Eff. (%) |
| --- | --- | --- | --- | --- |
| 1523.00 | 0.21 | 0.38 | 1.38 | 84.61 |
| 1540.00 | 2.31 | 4.19 | 15.49 | 82.09 |
| 1551.00 | 4.13 | 7.49 | 29.14 | 80.91 |
| 1519.00 | 6.24 | 11.33 | 44.66 | 81.66 |
| 1509.00 | 8.16 | 14.82 | 64.46 | 81.52 |

HBP,HJW & HGas

| Speed (rpm) | Load (kg) | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- |
| 1523.00 | 0.21 | 1.16 | 39.48 | 25.59 | 33.77 |
| 1540.00 | 2.31 | 8.94 | 34.92 | 26.10 | 30.03 |
| 1551.00 | 4.13 | 16.11 | 41.44 | 29.89 | 12.56 |
| 1519.00 | 6.24 | 22.16 | 42.65 | 31.56 | 3.64 |
| 1509.00 | 8.16 | 22.39 | 37.14 | 28.56 | 11.91 |

Observation Data

| Speed (rpm) | Load (kg) | Comp Ratio | T1 (deg C) | T2 (deg C) | T3 (deg C) | T4 (deg C) | T5 (deg C) | T6 (deg C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1523 | 0.21 | 15.00 | 25.56 | 31.48 | 25.57 | 28.39 | 158.78 | 133.10 |
| 1540 | 2.31 | 15.00 | 25.64 | 33.20 | 25.65 | 30.79 | 223.51 | 187.07 |
| 1551 | 4.13 | 15.00 | 25.62 | 34.59 | 25.61 | 33.12 | 253.65 | 214.33 |
| 1519 | 6.24 | 15.00 | 25.66 | 35.60 | 25.66 | 34.26 | 287.24 | 239.66 |
| 1509 | 8.16 | 15.00 | 25.73 | 36.87 | 25.73 | 36.35 | 330.20 | 273.19 |

Observation Data

| Air (mmWC) | Fuel (cc/min) | WaterFlow Engine (lph) | WaterFlow Cal (lph) |
| --- | --- | --- | --- |
| 84.96 | 9.00 | 300 | 100 |
| 81.76 | 13.00 | 300 | 100 |
| 80.58 | 13.00 | 300 | 100 |
| 78.73 | 14.00 | 300 | 100 |
| 77.43 | 18.00 | 300 | 100 |

Result Data

| Torque (Nm) | BP (kW) | FP (kW) | IP (kW) | BMEP (bar) | IMEP (bar) | BTHE (%) | ITHE (%) | Mech Eff. (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.38 | 0.06 | 4.33 | 4.39 | 0.07 | 5.24 | 1.16 | 84.05 | 1.38 |
| 4.19 | 0.68 | 3.69 | 4.36 | 0.80 | 5.14 | 8.94 | 57.74 | 15.49 |
| 7.49 | 1.22 | 2.96 | 4.18 | 1.42 | 4.88 | 16.11 | 55.29 | 29.14 |
| 11.33 | 1.80 | 2.23 | 4.03 | 2.15 | 4.82 | 22.16 | 49.61 | 44.66 |
| 14.82 | 2.34 | 1.29 | 3.63 | 2.81 | 4.37 | 22.39 | 34.73 | 64.46 |

Result Data

| Air Flow (kg/h) | Fuel Flow (kg/h) | SFC (kg/kWh) | Vol Eff. (%) | A/F Ratio | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30.02 | 0.45 | 7.37 | 84.61 | 66.97 | 1.16 | 39.48 | 25.59 | 33.77 |
| 29.45 | 0.65 | 0.96 | 82.09 | 45.49 | 8.94 | 34.92 | 26.10 | 30.03 |
| 29.23 | 0.65 | 0.53 | 80.91 | 45.16 | 16.11 | 41.44 | 29.89 | 12.56 |
| 28.90 | 0.70 | 0.39 | 81.66 | 41.45 | 22.16 | 42.65 | 31.56 | 3.64 |
| 28.66 | 0.90 | 0.38 | 81.52 | 31.97 | 22.39 | 37.14 | 28.56 | 11.91 |