ICEngineSoft Test Report

Report Date : Fri, Jan 27, 2017 , Time :  1:20:13 PM

Organization : Organisation, Operator : Operator

Data File : T9.COMPUTER.xlsx, Last Modified Date: Fri, Jan 27, 2017,

Config. File : VCR Diesel Engine.xls

Engine Details :

ICEngine set up under test is Research Diesel having power 3.50 kW @ 1500 rpm which is 1 Cylinder, Four stroke , Constant Speed, Water Cooled, Diesel Engine, with Cylinder Bore 87.50(mm), Stroke Length 110.00(mm), Connecting Rod length 234.00(mm), Compression Ratio 18.00, Swept volume 661.45 (cc)

Combustion Parameters :

Specific Gas Const (kJ/kgK) : 1.00, Air Density (kg/m^3) : 1.17, Adiabatic Index : 1.41, Polytrophic Index : -0.00, Number Of Cycles : 10, Cylinder Pressure Referance :  2, Smoothing 2, TDC Reference : 0

Performance Parameters :

Orifice Diameter (mm) : 20.00, Orifice Coeff. Of Discharge : 0.60, Dynamometer Arm Legnth (mm) : 185, Fuel Pipe dia (mm) : 12.40, Ambient Temp. (Deg C) : 27, Pulses Per revolution : 360, Fuel Type : Diesel, Fuel Density (Kg/m^3) : 830, Calorific Value Of Fuel (kj/kg) : 42000

IP, BP & FP

| Speed (rpm) | Load (kg) | IP (kW) | BP (kW) | FP (kW) |
| --- | --- | --- | --- | --- |
| 1537.00 | 2.31 | 5.89 | 0.67 | 5.21 |
| 1527.00 | 4.11 | 5.63 | 1.19 | 4.44 |
| 1517.00 | 6.19 | 6.26 | 1.79 | 4.47 |
| 1508.00 | 8.07 | 6.47 | 2.31 | 4.16 |

IMEP,BMEP & FMEP

| Speed (rpm) | Load (kg) | IMEP (bar) | BMEP (bar) | FMEP (bar) |
| --- | --- | --- | --- | --- |
| 1537.00 | 2.31 | 6.95 | 0.80 | 6.15 |
| 1527.00 | 4.11 | 6.69 | 1.42 | 5.27 |
| 1517.00 | 6.19 | 7.48 | 2.13 | 5.35 |
| 1508.00 | 8.07 | 7.79 | 2.78 | 5.01 |

Air &  Fuel Flow

| Speed (rpm) | Load (kg) | Air (mmWC) | Fuel (cc/min) |
| --- | --- | --- | --- |
| 1537.00 | 2.31 | 91.68 | 11.00 |
| 1527.00 | 4.11 | 89.42 | 13.00 |
| 1517.00 | 6.19 | 87.59 | 16.00 |
| 1508.00 | 8.07 | 85.05 | 17.00 |

Indicated & Brake Thermal Efficiency

| Speed (rpm) | Load (kg) | IThEff (%) | BThEff (%) |
| --- | --- | --- | --- |
| 1537.00 | 2.31 | 92.11 | 10.55 |
| 1527.00 | 4.11 | 74.56 | 15.79 |
| 1517.00 | 6.19 | 67.32 | 19.20 |
| 1508.00 | 8.07 | 65.54 | 23.41 |

SFC & Fuel Consumption

| Speed (rpm) | Load (kg) | SFC (kg/kWh) | Fuel (kg/h) |
| --- | --- | --- | --- |
| 1537.00 | 2.31 | 0.81 | 0.55 |
| 1527.00 | 4.11 | 0.54 | 0.65 |
| 1517.00 | 6.19 | 0.45 | 0.80 |
| 1508.00 | 8.07 | 0.37 | 0.85 |

TORQUE, Mechanical & Volmertic Efficiency

| Speed (rpm) | Load (Kg) | Torque (Nm) | Mech Eff. (%) | Vol Eff. (%) |
| --- | --- | --- | --- | --- |
| 1537.00 | 2.31 | 4.19 | 11.45 | 87.09 |
| 1527.00 | 4.11 | 7.46 | 21.17 | 86.57 |
| 1517.00 | 6.19 | 11.24 | 28.52 | 86.25 |
| 1508.00 | 8.07 | 14.64 | 35.72 | 85.50 |

HBP,HJW & HGas

| Speed (rpm) | Load (kg) | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- |
| 1537.00 | 2.31 | 10.55 | 48.19 | 31.44 | 9.82 |
| 1527.00 | 4.11 | 15.79 | 42.35 | 29.30 | 12.56 |
| 1517.00 | 6.19 | 19.20 | 37.58 | 27.25 | 15.97 |
| 1508.00 | 8.07 | 23.41 | 39.54 | 28.69 | 8.36 |

Observation Data

| Speed (rpm) | Load (kg) | Comp Ratio | T1 (deg C) | T2 (deg C) | T3 (deg C) | T4 (deg C) | T5 (deg C) | T6 (deg C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1537 | 2.31 | 18.00 | 26.14 | 34.97 | 26.14 | 32.17 | 216.96 | 187.98 |
| 1527 | 4.11 | 18.00 | 26.21 | 35.38 | 26.21 | 33.18 | 238.13 | 203.92 |
| 1517 | 6.19 | 18.00 | 26.26 | 36.27 | 26.24 | 34.85 | 269.99 | 227.91 |
| 1508 | 8.07 | 18.00 | 26.31 | 37.50 | 26.30 | 36.72 | 302.32 | 252.67 |

Observation Data

| Air (mmWC) | Fuel (cc/min) | WaterFlow Engine (lph) | WaterFlow Cal (lph) |
| --- | --- | --- | --- |
| 91.68 | 11.00 | 300 | 100 |
| 89.42 | 13.00 | 300 | 100 |
| 87.59 | 16.00 | 300 | 100 |
| 85.05 | 17.00 | 300 | 100 |

Result Data

| Torque (Nm) | BP (kW) | FP (kW) | IP (kW) | BMEP (bar) | IMEP (bar) | BTHE (%) | ITHE (%) | Mech Eff. (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.19 | 0.67 | 5.21 | 5.89 | 0.80 | 6.95 | 10.55 | 92.11 | 11.45 |
| 7.46 | 1.19 | 4.44 | 5.63 | 1.42 | 6.69 | 15.79 | 74.56 | 21.17 |
| 11.24 | 1.79 | 4.47 | 6.26 | 2.13 | 7.48 | 19.20 | 67.32 | 28.52 |
| 14.64 | 2.31 | 4.16 | 6.47 | 2.78 | 7.79 | 23.41 | 65.54 | 35.72 |

Result Data

| Air Flow (kg/h) | Fuel Flow (kg/h) | SFC (kg/kWh) | Vol Eff. (%) | A/F Ratio | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31.18 | 0.55 | 0.81 | 87.09 | 56.92 | 10.55 | 48.19 | 31.44 | 9.82 |
| 30.80 | 0.65 | 0.54 | 86.57 | 47.57 | 15.79 | 42.35 | 29.30 | 12.56 |
| 30.48 | 0.80 | 0.45 | 86.25 | 38.25 | 19.20 | 37.58 | 27.25 | 15.97 |
| 30.03 | 0.85 | 0.37 | 85.50 | 35.47 | 23.41 | 39.54 | 28.69 | 8.36 |