ICEngineSoft Test Report

Report Date : Wed, Jan 23, 2019 , Time :  3:17:50 PM

Organization : Organisation, Operator : Operator

Data File : T3 CR 16.xlsx, Last Modified Date: Wed, Jan 23, 2019,

Config. File : VCR Diesel Engine.xls

Engine Details :

ICEngine set up under test is Research Diesel having power 3.50 kW @ 1500 rpm which is 1 Cylinder, Four stroke , Constant Speed, Water Cooled, Diesel Engine, with Cylinder Bore 87.50(mm), Stroke Length 110.00(mm), Connecting Rod length 234.00(mm), Compression Ratio 16.00, Swept volume 661.45 (cc)

Combustion Parameters :

Specific Gas Const (kJ/kgK) : 1.00, Air Density (kg/m^3) : 1.17, Adiabatic Index : 1.41, Polytrophic Index : 0.51, Number Of Cycles : 10, Cylinder Pressure Referance :  4, Smoothing 2, TDC Reference : 0

Performance Parameters :

Orifice Diameter (mm) : 20.00, Orifice Coeff. Of Discharge : 0.60, Dynamometer Arm Legnth (mm) : 185, Fuel Pipe dia (mm) : 12.40, Ambient Temp. (Deg C) : 27, Pulses Per revolution : 360, Fuel Type : Diesel, Fuel Density (Kg/m^3) : 830, Calorific Value Of Fuel (kj/kg) : 42000

IP, BP & FP

| Speed (rpm) | Load (kg) | IP (kW) | BP (kW) | FP (kW) |
| --- | --- | --- | --- | --- |
| 1578.00 | 0.25 | 14.80 | 0.08 | 14.73 |
| 1551.00 | 2.49 | 16.21 | 0.74 | 15.48 |
| 1529.00 | 4.21 | 16.43 | 1.22 | 15.21 |
| 1530.00 | 6.22 | 17.00 | 1.81 | 15.19 |
| 1520.00 | 8.23 | 17.52 | 2.38 | 15.15 |
| 1521.00 | 10.14 | 18.16 | 2.93 | 15.23 |

IMEP,BMEP & FMEP

| Speed (rpm) | Load (kg) | IMEP (bar) | BMEP (bar) | FMEP (bar) |
| --- | --- | --- | --- | --- |
| 1578.00 | 0.25 | 17.02 | 0.09 | 16.93 |
| 1551.00 | 2.49 | 18.96 | 0.86 | 18.10 |
| 1529.00 | 4.21 | 19.50 | 1.45 | 18.05 |
| 1530.00 | 6.22 | 20.16 | 2.14 | 18.01 |
| 1520.00 | 8.23 | 20.91 | 2.84 | 18.08 |
| 1521.00 | 10.14 | 21.66 | 3.50 | 18.17 |

Air &  Fuel Flow

| Speed (rpm) | Load (kg) | Air (mmWC) | Fuel (cc/min) |
| --- | --- | --- | --- |
| 1578.00 | 0.25 | 88.67 | 8.00 |
| 1551.00 | 2.49 | 82.21 | 13.00 |
| 1529.00 | 4.21 | 81.79 | 13.00 |
| 1530.00 | 6.22 | 79.39 | 14.00 |
| 1520.00 | 8.23 | 77.07 | 17.00 |
| 1521.00 | 10.14 | 75.00 | 19.00 |

Indicated & Brake Thermal Efficiency

| Speed (rpm) | Load (kg) | IThEff (%) | BThEff (%) |
| --- | --- | --- | --- |
| 1578.00 | 0.25 | 318.50 | 1.62 |
| 1551.00 | 2.49 | 214.64 | 9.73 |
| 1529.00 | 4.21 | 217.57 | 16.20 |
| 1530.00 | 6.22 | 209.00 | 22.23 |
| 1520.00 | 8.23 | 177.40 | 24.06 |
| 1521.00 | 10.14 | 164.52 | 26.55 |

SFC & Fuel Consumption

| Speed (rpm) | Load (kg) | SFC (kg/kWh) | Fuel (kg/h) |
| --- | --- | --- | --- |
| 1578.00 | 0.25 | 5.28 | 0.40 |
| 1551.00 | 2.49 | 0.88 | 0.65 |
| 1529.00 | 4.21 | 0.53 | 0.65 |
| 1530.00 | 6.22 | 0.39 | 0.70 |
| 1520.00 | 8.23 | 0.36 | 0.85 |
| 1521.00 | 10.14 | 0.32 | 0.95 |

TORQUE, Mechanical & Volmertic Efficiency

| Speed (rpm) | Load (Kg) | Torque (Nm) | Mech Eff. (%) | Vol Eff. (%) |
| --- | --- | --- | --- | --- |
| 1578.00 | 0.25 | 0.46 | 0.51 | 83.43 |
| 1551.00 | 2.49 | 4.53 | 4.54 | 81.73 |
| 1529.00 | 4.21 | 7.64 | 7.45 | 82.69 |
| 1530.00 | 6.22 | 11.28 | 10.63 | 81.41 |
| 1520.00 | 8.23 | 14.93 | 13.56 | 80.74 |
| 1521.00 | 10.14 | 18.40 | 16.14 | 79.60 |

HBP,HJW & HGas

| Speed (rpm) | Load (kg) | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- |
| 1578.00 | 0.25 | 1.62 | 15.37 | 25.13 | 57.87 |
| 1551.00 | 2.49 | 9.73 | 9.45 | 24.07 | 56.75 |
| 1529.00 | 4.21 | 16.20 | 9.44 | 26.69 | 47.67 |
| 1530.00 | 6.22 | 22.23 | 8.79 | 28.10 | 40.88 |
| 1520.00 | 8.23 | 24.06 | 7.25 | 26.60 | 42.08 |
| 1521.00 | 10.14 | 26.55 | 6.48 | 26.93 | 40.04 |

Observation Data

| Speed (rpm) | Load (kg) | Comp Ratio | T1 (deg C) | T2 (deg C) | T3 (deg C) | T4 (deg C) | T5 (deg C) | T6 (deg C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1578 | 0.25 | 16.00 | 21.59 | 24.66 | 21.59 | 22.24 | 150.06 | 126.28 |
| 1551 | 2.49 | 16.00 | 21.61 | 24.68 | 21.61 | 24.73 | 207.73 | 172.80 |
| 1529 | 4.21 | 16.00 | 21.61 | 24.68 | 21.62 | 26.80 | 227.94 | 191.43 |
| 1530 | 6.22 | 16.00 | 21.62 | 24.69 | 21.62 | 28.59 | 257.77 | 214.92 |
| 1520 | 8.23 | 16.00 | 21.62 | 24.70 | 21.62 | 30.88 | 294.81 | 243.73 |
| 1521 | 10.14 | 16.00 | 21.63 | 24.71 | 21.63 | 33.63 | 332.97 | 273.48 |

Observation Data

| Air (mmWC) | Fuel (cc/min) | WaterFlow Engine (lph) | WaterFlow Cal (lph) |
| --- | --- | --- | --- |
| 88.67 | 8.00 | 200 | 100 |
| 82.21 | 13.00 | 200 | 100 |
| 81.79 | 13.00 | 200 | 100 |
| 79.39 | 14.00 | 200 | 100 |
| 77.07 | 17.00 | 200 | 100 |
| 75.00 | 19.00 | 200 | 100 |

Result Data

| Torque (Nm) | BP (kW) | FP (kW) | IP (kW) | BMEP (bar) | IMEP (bar) | BTHE (%) | ITHE (%) | Mech Eff. (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.46 | 0.08 | 14.73 | 14.80 | 0.09 | 17.02 | 1.62 | 318.50 | 0.51 |
| 4.53 | 0.74 | 15.48 | 16.21 | 0.86 | 18.96 | 9.73 | 214.64 | 4.54 |
| 7.64 | 1.22 | 15.21 | 16.43 | 1.45 | 19.50 | 16.20 | 217.57 | 7.45 |
| 11.28 | 1.81 | 15.19 | 17.00 | 2.14 | 20.16 | 22.23 | 209.00 | 10.63 |
| 14.93 | 2.38 | 15.15 | 17.52 | 2.84 | 20.91 | 24.06 | 177.40 | 13.56 |
| 18.40 | 2.93 | 15.23 | 18.16 | 3.50 | 21.66 | 26.55 | 164.52 | 16.14 |

Result Data

| Air Flow (kg/h) | Fuel Flow (kg/h) | SFC (kg/kWh) | Vol Eff. (%) | A/F Ratio | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30.67 | 0.40 | 5.28 | 83.43 | 76.97 | 1.62 | 15.37 | 25.13 | 57.87 |
| 29.53 | 0.65 | 0.88 | 81.73 | 45.61 | 9.73 | 9.45 | 24.07 | 56.75 |
| 29.45 | 0.65 | 0.53 | 82.69 | 45.49 | 16.20 | 9.44 | 26.69 | 47.67 |
| 29.02 | 0.70 | 0.39 | 81.41 | 41.62 | 22.23 | 8.79 | 28.10 | 40.88 |
| 28.59 | 0.85 | 0.36 | 80.74 | 33.77 | 24.06 | 7.25 | 26.60 | 42.08 |
| 28.20 | 0.95 | 0.32 | 79.60 | 29.81 | 26.55 | 6.48 | 26.93 | 40.04 |