ICEngineSoft Test Report

Report Date : Sat, Feb 02, 2019 , Time :  10:57:08 AM

Organization : Organisation, Operator : Operator

Data File : T5 CR 17.xlsx, Last Modified Date: Sat, Feb 02, 2019,

Config. File : VCR Diesel Engine.xls

Engine Details :

ICEngine set up under test is Research Diesel having power 3.50 kW @ 1500 rpm which is 1 Cylinder, Four stroke , Constant Speed, Water Cooled, Diesel Engine, with Cylinder Bore 87.50(mm), Stroke Length 110.00(mm), Connecting Rod length 234.00(mm), Compression Ratio 17.00, Swept volume 661.45 (cc)

Combustion Parameters :

Specific Gas Const (kJ/kgK) : 1.00, Air Density (kg/m^3) : 1.17, Adiabatic Index : 1.41, Polytrophic Index : -0.00, Number Of Cycles : 10, Cylinder Pressure Referance :  0, Smoothing 2, TDC Reference : 0

Performance Parameters :

Orifice Diameter (mm) : 20.00, Orifice Coeff. Of Discharge : 0.60, Dynamometer Arm Legnth (mm) : 185, Fuel Pipe dia (mm) : 12.40, Ambient Temp. (Deg C) : 27, Pulses Per revolution : 360, Fuel Type : Diesel, Fuel Density (Kg/m^3) : 830, Calorific Value Of Fuel (kj/kg) : 42000

IP, BP & FP

| Speed (rpm) | Load (kg) | IP (kW) | BP (kW) | FP (kW) |
| --- | --- | --- | --- | --- |
| 1555.00 | 0.30 | 14.94 | 0.09 | 14.85 |
| 1548.00 | 2.31 | 16.26 | 0.68 | 15.58 |
| 1537.00 | 4.42 | 17.28 | 1.29 | 15.98 |
| 1527.00 | 6.13 | 17.90 | 1.78 | 16.12 |
| 1519.00 | 8.25 | 18.50 | 2.38 | 16.12 |
| 1529.00 | 10.25 | 19.21 | 2.98 | 16.23 |

IMEP,BMEP & FMEP

| Speed (rpm) | Load (kg) | IMEP (bar) | BMEP (bar) | FMEP (bar) |
| --- | --- | --- | --- | --- |
| 1555.00 | 0.30 | 17.43 | 0.10 | 17.33 |
| 1548.00 | 2.31 | 19.06 | 0.80 | 18.26 |
| 1537.00 | 4.42 | 20.39 | 1.52 | 18.87 |
| 1527.00 | 6.13 | 21.26 | 2.11 | 19.15 |
| 1519.00 | 8.25 | 22.10 | 2.84 | 19.25 |
| 1529.00 | 10.25 | 22.79 | 3.53 | 19.26 |

Air &  Fuel Flow

| Speed (rpm) | Load (kg) | Air (mmWC) | Fuel (cc/min) |
| --- | --- | --- | --- |
| 1555.00 | 0.30 | 85.38 | 7.00 |
| 1548.00 | 2.31 | 82.67 | 11.00 |
| 1537.00 | 4.42 | 81.17 | 13.00 |
| 1527.00 | 6.13 | 80.28 | 12.00 |
| 1519.00 | 8.25 | 77.19 | 15.00 |
| 1529.00 | 10.25 | 75.00 | 17.00 |

Indicated & Brake Thermal Efficiency

| Speed (rpm) | Load (kg) | IThEff (%) | BThEff (%) |
| --- | --- | --- | --- |
| 1555.00 | 0.30 | 367.34 | 2.20 |
| 1548.00 | 2.31 | 254.43 | 10.66 |
| 1537.00 | 4.42 | 228.73 | 17.10 |
| 1527.00 | 6.13 | 256.68 | 25.50 |
| 1519.00 | 8.25 | 212.29 | 27.33 |
| 1529.00 | 10.25 | 194.46 | 30.15 |

SFC & Fuel Consumption

| Speed (rpm) | Load (kg) | SFC (kg/kWh) | Fuel (kg/h) |
| --- | --- | --- | --- |
| 1555.00 | 0.30 | 3.90 | 0.35 |
| 1548.00 | 2.31 | 0.80 | 0.55 |
| 1537.00 | 4.42 | 0.50 | 0.65 |
| 1527.00 | 6.13 | 0.34 | 0.60 |
| 1519.00 | 8.25 | 0.31 | 0.75 |
| 1529.00 | 10.25 | 0.28 | 0.85 |

TORQUE, Mechanical & Volmertic Efficiency

| Speed (rpm) | Load (Kg) | Torque (Nm) | Mech Eff. (%) | Vol Eff. (%) |
| --- | --- | --- | --- | --- |
| 1555.00 | 0.30 | 0.55 | 0.60 | 83.08 |
| 1548.00 | 2.31 | 4.20 | 4.19 | 82.12 |
| 1537.00 | 4.42 | 8.03 | 7.48 | 81.95 |
| 1527.00 | 6.13 | 11.12 | 9.93 | 82.03 |
| 1519.00 | 8.25 | 14.97 | 12.87 | 80.86 |
| 1529.00 | 10.25 | 18.60 | 15.51 | 79.19 |

HBP,HJW & HGas

| Speed (rpm) | Load (kg) | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- |
| 1555.00 | 0.30 | 2.20 | 17.51 | 28.16 | 52.13 |
| 1548.00 | 2.31 | 10.66 | 11.14 | 25.12 | 53.09 |
| 1537.00 | 4.42 | 17.10 | 9.42 | 24.76 | 48.71 |
| 1527.00 | 6.13 | 25.50 | 10.23 | 30.18 | 34.09 |
| 1519.00 | 8.25 | 27.33 | 8.19 | 27.75 | 36.72 |
| 1529.00 | 10.25 | 30.15 | 7.22 | 28.18 | 34.44 |

Observation Data

| Speed (rpm) | Load (kg) | Comp Ratio | T1 (deg C) | T2 (deg C) | T3 (deg C) | T4 (deg C) | T5 (deg C) | T6 (deg C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1555 | 0.30 | 17.00 | 21.61 | 24.67 | 21.61 | 23.79 | 139.89 | 121.72 |
| 1548 | 2.31 | 17.00 | 21.62 | 24.68 | 21.62 | 25.41 | 186.68 | 157.92 |
| 1537 | 4.42 | 17.00 | 21.61 | 24.67 | 21.62 | 26.56 | 214.08 | 179.89 |
| 1527 | 6.13 | 17.00 | 21.62 | 24.69 | 21.62 | 27.73 | 238.99 | 200.62 |
| 1519 | 8.25 | 17.00 | 21.62 | 24.69 | 21.62 | 29.16 | 274.15 | 226.78 |
| 1529 | 10.25 | 17.00 | 21.61 | 24.68 | 21.63 | 31.11 | 314.42 | 258.31 |

Observation Data

| Air (mmWC) | Fuel (cc/min) | WaterFlow Engine (lph) | WaterFlow Cal (lph) |
| --- | --- | --- | --- |
| 85.38 | 7.00 | 200 | 100 |
| 82.67 | 11.00 | 200 | 100 |
| 81.17 | 13.00 | 200 | 100 |
| 80.28 | 12.00 | 200 | 100 |
| 77.19 | 15.00 | 200 | 100 |
| 75.00 | 17.00 | 200 | 100 |

Result Data

| Torque (Nm) | BP (kW) | FP (kW) | IP (kW) | BMEP (bar) | IMEP (bar) | BTHE (%) | ITHE (%) | Mech Eff. (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.55 | 0.09 | 14.85 | 14.94 | 0.10 | 17.43 | 2.20 | 367.34 | 0.60 |
| 4.20 | 0.68 | 15.58 | 16.26 | 0.80 | 19.06 | 10.66 | 254.43 | 4.19 |
| 8.03 | 1.29 | 15.98 | 17.28 | 1.52 | 20.39 | 17.10 | 228.73 | 7.48 |
| 11.12 | 1.78 | 16.12 | 17.90 | 2.11 | 21.26 | 25.50 | 256.68 | 9.93 |
| 14.97 | 2.38 | 16.12 | 18.50 | 2.84 | 22.10 | 27.33 | 212.29 | 12.87 |
| 18.60 | 2.98 | 16.23 | 19.21 | 3.53 | 22.79 | 30.15 | 194.46 | 15.51 |

Result Data

| Air Flow (kg/h) | Fuel Flow (kg/h) | SFC (kg/kWh) | Vol Eff. (%) | A/F Ratio | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30.09 | 0.35 | 3.90 | 83.08 | 86.32 | 2.20 | 17.51 | 28.16 | 52.13 |
| 29.61 | 0.55 | 0.80 | 82.12 | 54.05 | 10.66 | 11.14 | 25.12 | 53.09 |
| 29.34 | 0.65 | 0.50 | 81.95 | 45.32 | 17.10 | 9.42 | 24.76 | 48.71 |
| 29.18 | 0.60 | 0.34 | 82.03 | 48.83 | 25.50 | 10.23 | 30.18 | 34.09 |
| 28.61 | 0.75 | 0.31 | 80.86 | 38.30 | 27.33 | 8.19 | 27.75 | 36.72 |
| 28.20 | 0.85 | 0.28 | 79.19 | 33.31 | 30.15 | 7.22 | 28.18 | 34.44 |