ICEngineSoft Test Report

Report Date : Wed, Feb 07, 2018 , Time :  4:51:35 PM

Organization : Organisation, Operator : Operator

Data File : T4 CR 13.xlsx, Last Modified Date: Wed, Feb 07, 2018,

Config. File : VCR Diesel Engine.xls

Engine Details :

ICEngine set up under test is Research Diesel having power 3.50 kW @ 1500 rpm which is 1 Cylinder, Four stroke , Constant Speed, Water Cooled, Diesel Engine, with Cylinder Bore 87.50(mm), Stroke Length 110.00(mm), Connecting Rod length 234.00(mm), Compression Ratio 13.00, Swept volume 661.45 (cc)

Combustion Parameters :

Specific Gas Const (kJ/kgK) : 1.00, Air Density (kg/m^3) : 1.17, Adiabatic Index : 1.41, Polytrophic Index : -0.00, Number Of Cycles : 10, Cylinder Pressure Referance :  1, Smoothing 2, TDC Reference : 0

Performance Parameters :

Orifice Diameter (mm) : 20.00, Orifice Coeff. Of Discharge : 0.60, Dynamometer Arm Legnth (mm) : 185, Fuel Pipe dia (mm) : 12.40, Ambient Temp. (Deg C) : 27, Pulses Per revolution : 360, Fuel Type : Diesel, Fuel Density (Kg/m^3) : 830, Calorific Value Of Fuel (kj/kg) : 42000

IP, BP & FP

| Speed (rpm) | Load (kg) | IP (kW) | BP (kW) | FP (kW) |
| --- | --- | --- | --- | --- |
| 1582.00 | 0.29 | 6.15 | 0.09 | 6.07 |
| 1525.00 | 4.34 | 6.85 | 1.26 | 5.59 |
| 1518.00 | 6.04 | 7.30 | 1.74 | 5.56 |
| 1510.00 | 8.16 | 6.77 | 2.34 | 4.43 |
| 1505.00 | 10.06 | 7.07 | 2.88 | 4.19 |

IMEP,BMEP & FMEP

| Speed (rpm) | Load (kg) | IMEP (bar) | BMEP (bar) | FMEP (bar) |
| --- | --- | --- | --- | --- |
| 1582.00 | 0.29 | 7.06 | 0.10 | 6.96 |
| 1525.00 | 4.34 | 8.15 | 1.50 | 6.65 |
| 1518.00 | 6.04 | 8.72 | 2.08 | 6.64 |
| 1510.00 | 8.16 | 8.14 | 2.81 | 5.33 |
| 1505.00 | 10.06 | 8.52 | 3.47 | 5.05 |

Air &  Fuel Flow

| Speed (rpm) | Load (kg) | Air (mmWC) | Fuel (cc/min) |
| --- | --- | --- | --- |
| 1582.00 | 0.29 | 97.38 | 9.00 |
| 1525.00 | 4.34 | 90.44 | 13.00 |
| 1518.00 | 6.04 | 88.16 | 15.00 |
| 1510.00 | 8.16 | 85.78 | 18.00 |
| 1505.00 | 10.06 | 83.11 | 19.00 |

Indicated & Brake Thermal Efficiency

| Speed (rpm) | Load (kg) | IThEff (%) | BThEff (%) |
| --- | --- | --- | --- |
| 1582.00 | 0.29 | 117.70 | 1.65 |
| 1525.00 | 4.34 | 90.71 | 16.66 |
| 1518.00 | 6.04 | 83.77 | 20.01 |
| 1510.00 | 8.16 | 64.77 | 22.39 |
| 1505.00 | 10.06 | 64.05 | 26.07 |

SFC & Fuel Consumption

| Speed (rpm) | Load (kg) | SFC (kg/kWh) | Fuel (kg/h) |
| --- | --- | --- | --- |
| 1582.00 | 0.29 | 5.19 | 0.45 |
| 1525.00 | 4.34 | 0.51 | 0.65 |
| 1518.00 | 6.04 | 0.43 | 0.75 |
| 1510.00 | 8.16 | 0.38 | 0.90 |
| 1505.00 | 10.06 | 0.33 | 0.95 |

TORQUE, Mechanical & Volmertic Efficiency

| Speed (rpm) | Load (Kg) | Torque (Nm) | Mech Eff. (%) | Vol Eff. (%) |
| --- | --- | --- | --- | --- |
| 1582.00 | 0.29 | 0.52 | 1.40 | 87.21 |
| 1525.00 | 4.34 | 7.88 | 18.37 | 87.18 |
| 1518.00 | 6.04 | 10.97 | 23.88 | 86.47 |
| 1510.00 | 8.16 | 14.80 | 34.56 | 85.75 |
| 1505.00 | 10.06 | 18.26 | 40.70 | 84.69 |

HBP,HJW & HGas

| Speed (rpm) | Load (kg) | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- |
| 1582.00 | 0.29 | 1.65 | 572.22 | 32.56 | 0.00 |
| 1525.00 | 4.34 | 16.66 | 388.71 | 32.46 | 0.00 |
| 1518.00 | 6.04 | 20.01 | 343.55 | 31.75 | 0.00 |
| 1510.00 | 8.16 | 22.39 | 286.36 | 30.27 | 0.00 |
| 1505.00 | 10.06 | 26.07 | 271.44 | 31.98 | 0.00 |

Observation Data

| Speed (rpm) | Load (kg) | Comp Ratio | T1 (deg C) | T2 (deg C) | T3 (deg C) | T4 (deg C) | T5 (deg C) | T6 (deg C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1582 | 0.29 | 13.00 | 25.45 | 111.23 | 25.47 | 30.17 | 183.72 | 158.21 |
| 1525 | 4.34 | 13.00 | 25.41 | 109.58 | 25.45 | 37.39 | 259.59 | 219.68 |
| 1518 | 6.04 | 13.00 | 25.40 | 111.22 | 25.42 | 39.60 | 292.02 | 245.02 |
| 1510 | 8.16 | 13.00 | 25.38 | 111.23 | 25.39 | 43.51 | 332.79 | 276.08 |
| 1505 | 10.06 | 13.00 | 25.33 | 111.23 | 25.34 | 48.06 | 372.71 | 303.64 |

Observation Data

| Air (mmWC) | Fuel (cc/min) | WaterFlow Engine (lph) | WaterFlow Cal (lph) |
| --- | --- | --- | --- |
| 97.38 | 9.00 | 300 | 100 |
| 90.44 | 13.00 | 300 | 100 |
| 88.16 | 15.00 | 300 | 100 |
| 85.78 | 18.00 | 300 | 100 |
| 83.11 | 19.00 | 300 | 100 |

Result Data

| Torque (Nm) | BP (kW) | FP (kW) | IP (kW) | BMEP (bar) | IMEP (bar) | BTHE (%) | ITHE (%) | Mech Eff. (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.52 | 0.09 | 6.07 | 6.15 | 0.10 | 7.06 | 1.65 | 117.70 | 1.40 |
| 7.88 | 1.26 | 5.59 | 6.85 | 1.50 | 8.15 | 16.66 | 90.71 | 18.37 |
| 10.97 | 1.74 | 5.56 | 7.30 | 2.08 | 8.72 | 20.01 | 83.77 | 23.88 |
| 14.80 | 2.34 | 4.43 | 6.77 | 2.81 | 8.14 | 22.39 | 64.77 | 34.56 |
| 18.26 | 2.88 | 4.19 | 7.07 | 3.47 | 8.52 | 26.07 | 64.05 | 40.70 |

Result Data

| Air Flow (kg/h) | Fuel Flow (kg/h) | SFC (kg/kWh) | Vol Eff. (%) | A/F Ratio | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32.14 | 0.45 | 5.19 | 87.21 | 71.70 | 1.65 | 572.22 | 32.56 | 0.00 |
| 30.97 | 0.65 | 0.51 | 87.18 | 47.84 | 16.66 | 388.71 | 32.46 | 0.00 |
| 30.58 | 0.75 | 0.43 | 86.47 | 40.93 | 20.01 | 343.55 | 31.75 | 0.00 |
| 30.16 | 0.90 | 0.38 | 85.75 | 33.65 | 22.39 | 286.36 | 30.27 | 0.00 |
| 29.69 | 0.95 | 0.33 | 84.69 | 31.38 | 26.07 | 271.44 | 31.98 | 0.00 |