ICEngineSoft Test Report

Report Date : Tue, Jan 23, 2018 , Time :  11:16:14 AM

Organization : Organisation, Operator : Operator

Data File : T1.CR18.xlsx, Last Modified Date: Tue, Jan 23, 2018,

Config. File : VCR Diesel Engine.xls

Engine Details :

ICEngine set up under test is Research Diesel having power 3.50 kW @ 1500 rpm which is 1 Cylinder, Four stroke , Constant Speed, Water Cooled, Diesel Engine, with Cylinder Bore 87.50(mm), Stroke Length 110.00(mm), Connecting Rod length 234.00(mm), Compression Ratio 18.00, Swept volume 661.45 (cc)

Combustion Parameters :

Specific Gas Const (kJ/kgK) : 1.00, Air Density (kg/m^3) : 1.17, Adiabatic Index : 1.41, Polytrophic Index : -0.00, Number Of Cycles : 10, Cylinder Pressure Referance :  1, Smoothing 2, TDC Reference : 0

Performance Parameters :

Orifice Diameter (mm) : 20.00, Orifice Coeff. Of Discharge : 0.60, Dynamometer Arm Legnth (mm) : 185, Fuel Pipe dia (mm) : 12.40, Ambient Temp. (Deg C) : 27, Pulses Per revolution : 360, Fuel Type : Diesel, Fuel Density (Kg/m^3) : 830, Calorific Value Of Fuel (kj/kg) : 42000

IP, BP & FP

| Speed (rpm) | Load (kg) | IP (kW) | BP (kW) | FP (kW) |
| --- | --- | --- | --- | --- |
| 1532.00 | 0.29 | 4.03 | 0.09 | 3.94 |
| 1539.00 | 2.70 | 3.75 | 0.79 | 2.96 |
| 1536.00 | 4.23 | 3.58 | 1.23 | 2.35 |
| 1524.00 | 6.44 | 5.48 | 1.86 | 3.62 |
| 1518.00 | 7.87 | 4.68 | 2.27 | 2.41 |

IMEP,BMEP & FMEP

| Speed (rpm) | Load (kg) | IMEP (bar) | BMEP (bar) | FMEP (bar) |
| --- | --- | --- | --- | --- |
| 1532.00 | 0.29 | 4.77 | 0.10 | 4.67 |
| 1539.00 | 2.70 | 4.42 | 0.93 | 3.48 |
| 1536.00 | 4.23 | 4.23 | 1.46 | 2.77 |
| 1524.00 | 6.44 | 6.53 | 2.22 | 4.31 |
| 1518.00 | 7.87 | 5.59 | 2.71 | 2.88 |

Air &  Fuel Flow

| Speed (rpm) | Load (kg) | Air (mmWC) | Fuel (cc/min) |
| --- | --- | --- | --- |
| 1532.00 | 0.29 | 97.64 | 9.00 |
| 1539.00 | 2.70 | 92.72 | 13.00 |
| 1536.00 | 4.23 | 91.29 | 15.00 |
| 1524.00 | 6.44 | 88.87 | 16.00 |
| 1518.00 | 7.87 | 86.43 | 17.00 |

Indicated & Brake Thermal Efficiency

| Speed (rpm) | Load (kg) | IThEff (%) | BThEff (%) |
| --- | --- | --- | --- |
| 1532.00 | 0.29 | 77.00 | 1.63 |
| 1539.00 | 2.70 | 49.60 | 10.47 |
| 1536.00 | 4.23 | 41.08 | 14.15 |
| 1524.00 | 6.44 | 58.98 | 20.06 |
| 1518.00 | 7.87 | 47.37 | 22.98 |

SFC & Fuel Consumption

| Speed (rpm) | Load (kg) | SFC (kg/kWh) | Fuel (kg/h) |
| --- | --- | --- | --- |
| 1532.00 | 0.29 | 5.25 | 0.45 |
| 1539.00 | 2.70 | 0.82 | 0.65 |
| 1536.00 | 4.23 | 0.61 | 0.75 |
| 1524.00 | 6.44 | 0.43 | 0.80 |
| 1518.00 | 7.87 | 0.37 | 0.85 |

TORQUE, Mechanical & Volmertic Efficiency

| Speed (rpm) | Load (Kg) | Torque (Nm) | Mech Eff. (%) | Vol Eff. (%) |
| --- | --- | --- | --- | --- |
| 1532.00 | 0.29 | 0.53 | 2.12 | 90.17 |
| 1539.00 | 2.70 | 4.91 | 21.11 | 87.47 |
| 1536.00 | 4.23 | 7.67 | 34.46 | 86.97 |
| 1524.00 | 6.44 | 11.68 | 34.01 | 86.48 |
| 1518.00 | 7.87 | 14.28 | 48.51 | 85.62 |

HBP,HJW & HGas

| Speed (rpm) | Load (kg) | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- |
| 1532.00 | 0.29 | 1.63 | 52.61 | 26.44 | 19.32 |
| 1539.00 | 2.70 | 10.47 | 41.32 | 26.31 | 21.90 |
| 1536.00 | 4.23 | 14.15 | 43.53 | 25.92 | 16.40 |
| 1524.00 | 6.44 | 20.06 | 44.20 | 27.76 | 7.97 |
| 1518.00 | 7.87 | 22.98 | 43.29 | 28.50 | 5.23 |

Observation Data

| Speed (rpm) | Load (kg) | Comp Ratio | T1 (deg C) | T2 (deg C) | T3 (deg C) | T4 (deg C) | T5 (deg C) | T6 (deg C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1532 | 0.29 | 18.00 | 18.79 | 26.68 | 18.80 | 23.62 | 154.10 | 127.83 |
| 1539 | 2.70 | 18.00 | 18.95 | 27.90 | 18.94 | 25.90 | 213.28 | 174.35 |
| 1536 | 4.23 | 18.00 | 19.17 | 30.05 | 19.18 | 30.38 | 239.68 | 193.19 |
| 1524 | 6.44 | 18.00 | 19.29 | 31.07 | 19.29 | 32.52 | 272.81 | 207.54 |
| 1518 | 7.87 | 18.00 | 19.35 | 31.60 | 19.34 | 34.45 | 298.32 | 241.57 |

Observation Data

| Air (mmWC) | Fuel (cc/min) | WaterFlow Engine (lph) | WaterFlow Cal (lph) |
| --- | --- | --- | --- |
| 97.64 | 9.00 | 300 | 100 |
| 92.72 | 13.00 | 300 | 100 |
| 91.29 | 15.00 | 300 | 100 |
| 88.87 | 16.00 | 300 | 100 |
| 86.43 | 17.00 | 300 | 100 |

Result Data

| Torque (Nm) | BP (kW) | FP (kW) | IP (kW) | BMEP (bar) | IMEP (bar) | BTHE (%) | ITHE (%) | Mech Eff. (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.53 | 0.09 | 3.94 | 4.03 | 0.10 | 4.77 | 1.63 | 77.00 | 2.12 |
| 4.91 | 0.79 | 2.96 | 3.75 | 0.93 | 4.42 | 10.47 | 49.60 | 21.11 |
| 7.67 | 1.23 | 2.35 | 3.58 | 1.46 | 4.23 | 14.15 | 41.08 | 34.46 |
| 11.68 | 1.86 | 3.62 | 5.48 | 2.22 | 6.53 | 20.06 | 58.98 | 34.01 |
| 14.28 | 2.27 | 2.41 | 4.68 | 2.71 | 5.59 | 22.98 | 47.37 | 48.51 |

Result Data

| Air Flow (kg/h) | Fuel Flow (kg/h) | SFC (kg/kWh) | Vol Eff. (%) | A/F Ratio | HBP (%) | HJW (%) | HGas (%) | HRad (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32.18 | 0.45 | 5.25 | 90.17 | 71.80 | 1.63 | 52.61 | 26.44 | 19.32 |
| 31.36 | 0.65 | 0.82 | 87.47 | 48.44 | 10.47 | 41.32 | 26.31 | 21.90 |
| 31.12 | 0.75 | 0.61 | 86.97 | 41.65 | 14.15 | 43.53 | 25.92 | 16.40 |
| 30.70 | 0.80 | 0.43 | 86.48 | 38.53 | 20.06 | 44.20 | 27.76 | 7.97 |
| 30.28 | 0.85 | 0.37 | 85.62 | 35.76 | 22.98 | 43.29 | 28.50 | 5.23 |